Signals and Transforms February 2023

G. Meinsma∗

H. G. ter Morsche∗∗

A.A. Stoorvogel∗

H. Zwart∗

∗ University of Twente

∗∗ University of Eindhoven

ii

Contents

1 Introduction to Banach and Hilbert Space 3 1.1 Module 2 summary: normed vector space & convergence . . . . . . . . . . . . . 4 1.2 Banach space . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9 1.3 Module 2 summary: inner product . . . . . . . . . . . . . . . . . . . . . . . . . . . 15 1.4 Orthogonal projection . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21 1.5 Hilbert space . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27 1.6 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

2 Introduction to Signals and Convolutions 43 2.1 Continuous-time and discrete-time signals . . . . . . . . . . . . . . . . . . . . . . 43 2.2 Review of complex numbers . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 44 2.3 Complex valued signals . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48 2.4 Periodic signals . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 49 2.5 Standard signals . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50 2.6 Energy and power . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 52 2.7 Convolutions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 53 2.8 The delta function . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 55 2.9 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 60

3 Fourier Series 65 3.1 The fundamental theorem of Fourier series . . . . . . . . . . . . . . . . . . . . . . 66 3.2 Pointwise convergence of Fourier series . . . . . . . . . . . . . . . . . . . . . . . . 70 3.3 Real Fourier series . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 73 3.4 Fourier series properties . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 76 3.5 Convolution and Parseval’s theorem . . . . . . . . . . . . . . . . . . . . . . . . . . 78 3.6 Gibbs phenomenon . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 81 3.7 Applications . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 83 3.8 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 89

4 Fourier Transform 95 4.1 The Fourier integral theorem . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 95 4.2 Fourier transform properties . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 100 4.3 Examples . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 103 4.4 Convolution and correlation . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 107 4.5 Fourier transforms with delta functions . . . . . . . . . . . . . . . . . . . . . . . . 109 4.6 Applications . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 111 4.7 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 117

5 Laplace Transform 121 5.1 Laplace transform . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 122

iii

5.2 Signals with delta function components . . . . . . . . . . . . . . . . . . . . . . . . 124 5.3 Properties of the Laplace transform . . . . . . . . . . . . . . . . . . . . . . . . . . 125 5.4 Examples . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 127 5.5 Limit behavior . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 130 5.6 Convolution in Laplace domain . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 132 5.7 Applications . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 133 5.8 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

A Appendices 143 A.1 Proofs . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143 A.1.1 Chapter 2 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143 A.1.2 Chapter 3 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 144 A.2 Bounded Linear Operators and Contractions . . . . . . . . . . . . . . . . . . . . . 147 A.2.1 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 152 A.3 Ordinary Differential Equations (ODEs) . . . . . . . . . . . . . . . . . . . . . . . . 153 A.3.1 Particular solutions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 155 A.3.2 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 157 A.4 Partial Fraction Expansion . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 158 A.4.1 If Q(s)/P(s) is strictly proper . . . . . . . . . . . . . . . . . . . . . . . . . . 159 A.4.2 If Q(s)/P(s) is proper . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 161 A.4.3 Complex poles . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 162 A.4.4 Exercises . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163 A.5 Complex Integration . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163 A.5.1 Three examples . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 164 A.6 Matlab & Python . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165 A.7 Generalized functions (UNDER CONSTRUCTION) . . . . . . . . . . . . . . . . . 167 A.7.1 Introduction to generalized functions . . . . . . . . . . . . . . . . . . . . 167 A.7.2 Generalized derivatives . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 169 A.7.3 Generalized Fourier transforms . . . . . . . . . . . . . . . . . . . . . . . . 170

B Answers to some exercises 173 B.1 Chapter 1 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 173 B.2 Chapter 2 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 177 B.3 Chapter 3 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 179 B.4 Chapter 4 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 183 B.5 Chapter 5 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 185 B.6 Appendix A.3 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 186 B.7 Appendix A.4 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 187

Index 189 iv

Why this course?

In 1807 Jean Baptiste Joseph Fourier, in his desire to determine the heat distribution in a metal plate, came up with the bold idea that every function can be written as a sum of harmonic functions. Understandably his claim was met with scepticism, especially because Fourier did not provide a proof of his claim. Several years later (still within Fourier’s lifetime) Niels Henrik Abel settled the issue with a proper proof: Fourier was right after all, and this sum of harmonic functions since then carries his name: the Fourier series.

Nowadays it is difficult to imagine a world without Fourier series. Trillions of Fourier se ries are calculated routinely every second. For instance your laptop computes 250000 such series for every second that it is talking wirelessly to your modem, and your modem does the same. Ever wondered what JPEG files are? They are essentially just a collection of Fourier co efficients that your computer (by computing a Fourier series) can turn into a picture. Fourier is everywhere in audio and video processing (such as MP3, MPEG, et cetera) and the digital revolution would have been way less succesfull if it were not for the Fourier series and the discovery of the “fast fourier transform” which is an algorithm that can compute Fourier se ries very efficiently.

Fourier analysis is a standard tool in many engineering sciences. It offers an alternative representation of “signals” and “systems” providing lots of insight, and the above mentioned set of applications are just a few of them. Fourier analysis plays a very important role in signal processing applications, it is the work-horse behind efficient algorithms for computation of products of very long integers, and is used to solve partial differential equations, and much more.

In this course we analyze the Fourier series in detail and we use it to solve a number of “signal processing” problems and — which is how it all started — to solve certain differential equations and partial differential equations. The course also introduces the Laplace trans form, which can be seen as an extension of the Fourier transform.

The material presented in Chapters 2–5 is quite common in engineering sciences. The first chapter, however, is of a different nature and might fit in an advanced course on linear algebra or a first course on functional analysis. It introduces Banach and Hilbert spaces and more, providing an abstract and general view of Fourier series.

1

2

Chapter 1

Introduction to Banach and Hilbert Space

This chapter assumes basis knowledge of linear algebra, in particular it assumes familiarity with real and complex vector space, with subspaces, basis and dimensions of finite dimen sional vector space, and with linear transformations defined on vector space. We make frequent use of the following notation:

N set of positive integers {1, 2, 3,...}

N0 set of nonnegative integers {0, 1, 2, 3,...}

Z set of integers {...,−2,−1, 0, 1, 2,...}

Q set of rational numbers

R set of real numbers

C set of complex numbers

Rnset of ordered n-tuples (u1,...,un) with uk ∈ R ∀k = 1, 2,...,n Cnset of ordered n-tuples (u1,...,un) with uk ∈ C ∀k = 1, 2,...,n

We always assume that Rnis equipped with the standard vector addition and scalar multipli cation, and this makes Rna real vector space. Likewise Cnis a complex vector space. Both are vector spaces of finite sequences. For vector spaces of infinite sequences – with similar vector addition and scalar multiplication – we use the notation

infinite sequence space {(u1,u2,...) | uk ∈ R∀k ∈ N}

(A;B) general sequence space {u : A → B} with A ⊆ Z. For instance  = (N;R)

finite {u : N → R | uk = 0 for finitely many k ∈ N} . It is a subset of .

p{u ∈ |∞k=1|uk |p < ∞}. Assumes 0 < p < ∞.

For p = ∞ it is defined as {u ∈ | supk∈N |uk | < ∞}. More on this later.

The above assume that A ⊆ Z. For arbitrary subsets A of R we use the following notation. Also these naturally are vector spaces.

F(A;B) Function space {f : A → B}, i.e. the set of all functions that map from A to B. For instance Cn = F({1,...,n};C) and  = F(N;R).

Typically, though, F is used for function spaces such as F([0, 1];R).

C (A;B) The set of functions f : A → B that are continuous. It is a subset of F(A;B). We assume A ⊆ R, and B = Rn or B = Cn.

L p(A;B) Set of functions f : A → B for which A|f (t)|p dt < ∞. It is a subset of F(A;B). Assumes 1 ≤ p < ∞ or p = ∞, and A ⊆ R, and B = Rn or B = Cn. More on this later.

3

1.1 Module 2 summary: normed vector space & convergence

A normed vector space loosely speaking is a vector space in which a “length” of a vector is available. This additional structure allows us to deal with convergence and optimal approxi mations within vector spaces. A “length” is called a norm if it has the following properties.

Definition 1.1.1 (norm). Let X be a real or complex vector space. A mapping  ·  from X to R is a norm if for all x, y ∈ X and all scalars α it satisfies the three axioms:

1. αx = |α|x, (positive homogeneous) 2. x + y ≤ x + y, (triangle inequality) 3. x > 0 for every x = 0. (positive definite)

For α = 0 the first axiom tells us that 0 = 0. So a norm x is zero if and only if x is the zero vector. A normed vector space is a vector space on which a norm is defined. Formally one should say “(X, · ) is a normed vector space” but we usually just say “X is a normed vector space” assuming that the choice of norm is clear from the problem at hand. Be aware, however, that a vector space can be equipped with different norms, see the next example.

(0,1)

x1 ≤ 1(1,0) (a)

(0,1)

x2 ≤ 1(1,0) (b)

(−1,1) (1,1) x∞ ≤ 1

(c)

FIGURE 1.1: The unit balls {x ∈ R2| xp ≤ 1} for p = 1, 2,∞.

Example 1.1.2 (Three different norms on R2). Consider R2.

1. The 1-norm on R2is defined as

x1 = |x1| +|x2|.

In the first quadrant — where x1 and x2 are nonnegative — the 1-norm is just the sum of the entries, x1 = x1 + x2. In the first quadrant therefore the norm is at most 1 iff x2 ≤ 1− x1, which is the region

(0,1)

{x ∈ R2| x1 ≥ 0,x2 ≥ 0,x1 + x2 ≤ 1}

(1,0)

Combined with the other three quadrants we get that the unit ball {x ∈ R2| x1 ≤ 1} is a polytope, see Fig. 1.1(a).

2. The Euclidean norm on R2, also known as the 2-norm, is defined as

x2:=

x21 + x22.

In this norm the unit ball {x ∈ R2| x2 ≤ 1} is the standard unit disc, see Fig. 1.1(b). 4

3. The max-norm, or ∞-norm on R2is defined as

x∞ = max(|x1|,|x2|).

Now in this norm the unit ball {x ∈ R2| x∞ ≤ 1} is a square with its axes parallel to the x1- and x2-axis, see Fig. 1.1(c).

In Exercise 1.1.2 we ask you to prove that all three are indeed norms.

The 1-norm is sometimes called the manhattan norm because in a rectangular street grid — which is common in US cities — the 1-norm x − y1 is the minimal Euclidean distance over the road required to travel from junction x to junction y, see Fig. 1.2.

y

x

FIGURE 1.2: Manhattan norm: all three routes from x to y are equally long: x − y1.

The triangle inequality x + y ≤ x + y roughly speaking says that, in any norm, trav eling from 0 to x + y via x or y can only mean a detour. Moving the y to the left-hand side of the inequality turns the triangle inequality into a statement that says that the length of any side in a triangle is at least the difference of that of the other two sides,

x + y − y ≤ x.

x + y x

y

This is sometimes called the reverse triangle inequality, and it is commonly formulated in terms of z = x + y as:

Lemma 1.1.3 (Reverse triangle inequality). |z − y| ≤ z − y.

The reverse triangle inequality demonstrates that when two vectors z and y are “close” then their norms are “close” as well:

Lemma 1.1.4 (Norms are continuous). Every norm ·  : X → R is uniformly continuous. Proof. Recall that a mapping A : X → R is continuous at x0 ∈ X if for every  > 0 there is a

> 0 such that x − x0 < δ implies |A(x)− A(x0)| < . For A :=  ·  we can simply take δ =

δ

because, by the reverse triangle inequality, |A(x)− A(x0)| = |x − x0| ≤ x − x0. This works for every x0 ∈ X. (Continuity is uniform if we can choose δ independent from x0; that is the case here.)

This is a very useful property, and we will exploit it later (be patient).

Example 1.1.5. Consider finite, which is the vector space of infinite sequences f = (f1, f2, f3,...) of real numbers, but with only finitely nonzero entries, so

finite = {f : N → R | fk = 0 for finitely many k ∈ N}.

5

On this space the 1-norm defined as ∞

f 1:=

k=1

|fk | (1.1)

is a norm (see Exercise 1.2).

Implicit in the definition of norm is that the norm is well defined (exists) for every element of the space. Hence on  — which we use to denote the vector space of all real-valued infinite sequences — the above 1-norm is not a norm because (1.1) is not convergent for the infinite sequence f = (1, 1, 1,...) ∈ .

Example 1.1.6 (Continuous functions in max-norm). Let a,b ∈ R and a < b. The standard norm on the vector space C ([a,b];R) of continuous functions on the real interval [a,b] is the max-norm, also known as ∞-norm, defined as

f ∞ = max

t∈[a,b]|f (t)|.

We now verify that this indeed satisfies the three axioms of norm, including existence:

0. Every continuous function on a finite interval is bounded and has a maximum, so f ∞ exists for all f ∈ C ([a,b];R).

1. For every scalar α we have

αf ∞ = max

t∈[a,b]|αf (t)| = max

t∈[a,b]|α||f (t)| = |α| max

t∈[a,b]|f (t)| = |α|f ∞.

2. The max norm inherits the triangle inequality from R: since for every p,q ∈ R we have that |p + q| ≤ |p| +|q|, we also have for every f , g ∈ C ([a,b];R) that

f + g ∞ = max

t∈[a,b]|f (t)+ g (t)|

≤ max

t∈[a,b]|f (t)| +|g (t)|

≤ max

t∈[a,b]|f (t)| + max

t∈[a,b]|g (t)| = f ∞ + g ∞.

3. If f is not the zero function then f (t0) = 0 for at least one t0 ∈ [a,b]. Now f ∞ ≥ |f (t0)| > 0.

In the literature the vector space C ([a,b];R) is often identified with the normed vector space (C ([a,b];R), · ∞). This is unfortunate since we might want to consider other norms on the space of continuous functions, for instance:

Example 1.1.7. Let a,b ∈ R and a < b. On C ([a,b];R) the function  · 1 : C ([a,b];R) → R defined as

b

f 1 =

|f (t)|dt (1.2) a

is a norm (see Exercise 1.3).

Notice that in this example the norm f 1 exists for every f ∈ C ([a,b];R). For the bigger set F([a,b];R) that is not case, and this is the reason we restricted attention to C ([a,b];R). However the space of continuous functions also has its drawbacks for this norm:

6

Example 1.1.8 (Limit does not exist in the space). Consider C ([−1, 1];R) and the 1-norm defined in (1.2). In this norm the sequence of functions

fn(t) =

 

0 t ∈ [−1, 0] nt t ∈ (0, 1n)

1

1 t ∈ [1n, 1]1/n

does not converge in the space C ([−1, 1];R) because no continuous function f exists for which limn→∞ fn − f 1 = 0. (Convince yourself of this; Exercise 1.13 might be helpfull here.) Never theless the sequence of functions do approach one another in this norm, in the sense that

sup

n≥N,m≥N

fn − fm1

goes to zero as N → ∞. This follows from the fact that for any n,m ≥ N we have

fn − fm1 =

1 −1

|fn(t)− fm(t)|dt =

1/min(n,m) 0

|fn(t)− fm(t)|dt ≤

1/N 0

1 dt =1N.

The above example demonstrates that there is a difference between converging sequences and sequences whose elements become closer and closer. The latter is called “Cauchy se quence,” which we discuss next. Incidentally this difference is not specific to vector space. It also shows up in sets like the rational numbers Q. Indeed, in Q we can construct sequences that approach one another in absolute value but that do not have a limit in the set of ra tional numbers. An example is the sequence of rational numbers {3, 3.1, 3.14, 3.141, ...} that converges to the non-rational π.

Convergence and the Cauchy sequence

Quite often a vector x can be shown to have some desirable property by creating a sequence of easier-to-work-with vectors {xn}n∈N and that converge to this vector x. It is for this reason that the concept of convergence plays an important role in analysis. We define two types of convergence:

Definition 1.1.9 (Cauchy sequence and convergent sequence). Let X be a normed vector space and let {xn}n∈N be a sequence in X (that is, xn ∈ X for every n ∈ N).

• We say that {xn}n∈N is a Cauchy sequence if xn − xm → 0 as n,m → ∞. This means

∀ > 0 ∃N

∈ N such that

xn − xm <  ∀n ≥ N,m ≥ N.

• We say that {xn}n∈N converges in X if an x ∈ X exists such that x − xn → 0 as n → ∞.

This means ∀ > 0 ∃N

> 0 such that

x − xn <  ∀n ≥ N.

In that case x is called the limit of the sequence.  7

If xn converges then its limit x is unique (Exercise 1.13(a)). We denote this limit x as x = limn→∞ xn. The Cauchy property can be expressed in terms of limits as well: a sequence xn is a Cauchy sequence (on some vector space with norm · ) if and only if

lim

N→∞sup n≥N,m≥N

xn − xm

= 0.

The term in between brackets is, so to say, the “maximal” distance between any two elements xn and xm in the “tail” of the sequence {xN ,xN+1,xN+2,...}. For real sequences this is de picted in Fig. 1.3. It can be shown that for sequences {xn} of real numbers the two notions are equivalent1. I.e. a real sequence converges iff it is a Cauchy sequence. Figure 1.3 makes this plausible.

xn

ǫ

Nǫn →

FIGURE 1.3: Cauchy criterion for real sequences xn.

Example 1.1.10 (Integral test for real-valued sequences). Consider the real sequence xn = 1+122 +132 +··· + 1n2 ∈ R. Now for every m ≥ n ≥ N we have

|xm − xn| = <

m

k=n+1 ∞

k=N+1

1

k2

1

k2

N +1

1/t2

t →

<

∞ N

1

t2dt

N +1

1/t2

t →

=−1t∞N=1N.

Therefore supn,m≥N |xm −xn| converges to zero as N → ∞, which shows that it is a Cauchy se quence. But for real sequences being Cauchy is equivalent to being convergent, so limn→∞ xn exists. The beauty and strength of Cauchy is that we do not need to know the limit of a real sequence in order for us to know that the limit exists. (Cliff hanger: later on in this course we will see that the limit is π2/6 = 1.644934..., see Example 3.5.5.)

While on R every Cauchy sequence converges, Example 1.1.8 demonstrates that Cauchy sequences on other normed vector spaces need not always converge. The converse does hold on every vector space:

Theorem 1.1.11 (Convergence implies Cauchy). Every convergent sequence is a Cauchy se quence.

Proof. If limn→∞ fn − f  = 0 then ∀ > 0 there exists N ∈ N such that fn − f  < /2 ∀n ≥ N. From the triangle inequality it now follows that fn − fm = (fn − f ) − (fm − f ) ≤ fn − f  + fm − f  < /2+/2 =  for every n,m ≥ N. So fn is a Cauchy sequence.

1See the course Analysis 1

8

1.2 Banach space

Normed sets on which every Cauchy sequence converges are so desirable that we give them a name:

Definition 1.2.1 (Complete set & Banach space). A normed set X is complete if every Cauchy sequence in X converges in X. A Banach space is a complete normed vector space.

In a Banach space, therefore, a sequence converges if and only if it is a Cauchy sequence. This is convenient because the Cauchy property is often easier to check as it does not require knowledge of the limit, see Example 1.1.10. This will be of great help in the final section of this chapter.

Over the years many vector spaces have been shown to be Banach spaces, but also many fail to be. In this course we will not worry about completeness proofs too much because the proofs are often technical. We simply list a couple in the remainder of this section, and we prove only a few.

Theorem 1.2.2 (Continuous functions with max norm). C ([a,b];R) is a Banach space in the max-norm · ∞.

Proof. Suppose fn is a Cauchy sequence. Then ∀ > 0 there is an N > 0 such that fn − fm∞ <  for all n,m ≥ N. Then at every t ∈ [a,b] we have

|fn(t)− fm(t)| ≤ fn − fm∞ <  ∀n,m ≥ N. (1.3)

So for every fixed t ∈ [a,b] the sequence of real numbers {fm(t)}m∈N is Cauchy. Since R (in the absolute value as norm) is complete, we have for every t that the sequence fm(t) of real numbers converges as m → ∞. Denote this limit as f (t). Letting m → ∞ in (1.3) shows that

|fn(t)− f (t)| ≤  ∀n ≥ N,

and that this N

does not depend on t. Hence fn − f ∞ → 0 as n → ∞. Remains to show that

this f is continuous. Fix an n ≥ N/3. By continuity of fn we have at each t that |fn(t)− fn(t + h)| < /3 for all h ∈ [−δt,δt ] for some small enough δt. For all such h there holds

|f (t +h)− f (t)| = |f (t +h)− fn(t +h)+ fn(t +h)− fn(t)+ fn(t)− f (t)|

≤ |f (t +h)− fn(t +h)| +|fn(t +h)− fn(t)| +|fn(t)− f (t)|

< /3+/3+/3 = .

So f is continuous.

Notice that C ([a,b];R) is not complete in the 1-norm (Example 1.1.8) thus completeness not only depends on the vector space, it also depends on the choice of norm. On finite di mensional space, however, it does not depend on the choice of norm:

Theorem 1.2.3 (Finite dimensional space). Every real or complex finite dimensional normed vector space is a Banach space.

Proof (idea only). Let S :={v1,..., vm} be a basis of the space. If fn is a Cauchy sequence then its coordinate vectors cn (with respect to basis S) is a Cauchy sequence in Rm (or Cm) in, for example, the Euclidean norm. This implies that each entry cn,i of these coordinate vectors is a Cauchy sequence in R (or C) with norm xi  = |xi|. By completeness of R each entry cn,i hence converges as n → ∞ to some c∞,i. The corresponding f :=c∞,1v1 +··· +c∞,mvm is well defined, and one can show that limn→∞ fn − f  = 0. So fn converges as n → ∞.

9

This theorem implies that R2is a Banach space in each of the three norms as considered in Example 1.1.2. On infinite dimensional vector space matters are more complicated. Let us have a look at the infinite sequence space  defined as

= {(v1, v2, v3,...)| vi ∈ R∀i ∈ N}.

On this set the 1-norm, 2-norm and ∞-norm, that we defined on Rn, become the series and supremum

∞

v1:=

i=1

|vi|

v2:=

∞

i=1

|vi|2

v∞ :=sup i≥1

|vi|.

These, however, are not norms on  for the simple reason that v1 and v2 are not always convergent, and v∞ does not always exist (i.e. might be ∞). For instance all three “norms” are not defined for the growing sequence

v = (1, 2, 3, 4, 5,...) ∈ .

The way out of this problem is as simple as it is elegant. Merely restricting the sequence space  to those elements that have finite norm will do the job, and the result is a Banach space:

Theorem 1.2.4 (Complete sequence spaces). All three sequence spaces

1:={v ∈  | v1 < ∞},

2:={v ∈  | v2 < ∞},

∞ :={v ∈  | v∞ < ∞}

are complete vector spaces in their respective norms, i.e. they are Banach spaces in their re spective norms. The same holds for the complex versions 1(N;C),2(N;C),∞(N;C).

Proof . We prove it only for 2, which is the most important case for our course. Exer cise 1.27 shows that 2is a vector space and that v2 is a norm on this vector space. The completeness proof we do here. This proof follows the standard steps in completeness proofs:

• assume vn is a Cauchy sequence,

• suggest a candidate limit w of vn,

• show that w is in the space (i.e. in 2),

• show that limn→∞ vn = w.

So assume that vn is Cauchy, and realize that each vn is itself a sequence, vn = (vn,1, vn,2, vn,3,...). For every fixed index k0 ∈ N, the entries {vn,k0}n∈N is a Cauchy sequence because

|vn,k0 − vm,k0| ≤

∞

k=1

|vn,k − vm,k |2 = vn − vm2 → 0 as n,m → ∞.

Since vn,k0is in R, and R is complete, we conclude that wk0:=limn→∞ vn,k0exists. As candi date limit w of vn we now propose w :=(w1,w2,w3,...). Next we show that this w is in 2.

10

Since vn is Cauchy, there is for every  > 0 an N ∈ N such that vn −vm2 <  for all n,m ≥ N. Now fix an L ∈ N and note that then

L

|vn,k − vm,k |2 < 2 ∀n,m ≥ N.

k=1

For m → ∞ this inequality says that Lk=1|vn,k − wk |2 ≤ 2for all n ≥ N. This holds for every L, and so as L → ∞ we find that

∞

k=1

|vn,k −wk |2 ≤ 2 ∀n ≥ N.

This shows that

vn −w2 ≤  ∀n ≥ N. (1.4) Hence vn −w ∈ 2for every n ≥ N. Since 2is a vector space, and vn ∈ 2, we see that w is in

2as well. Finally, (1.4) by definition means that w = limn→∞ vn.

Example 1.2.5 (Cauchy or not Cauchy). Consider the infinite sequence vn = (1, 12,13,...,1n, 0, 0,...) ∈ ,

depending on n ∈ N. For every n the sequence vn has only finitely many nonzero entries, so it has finite 1-norm, finite 2-norm and finite ∞-norm, and thus is in all three vector spaces

1,2and ∞. The sequence vn pointwise converges to the infinite sequence

w = (1, 12,13,14,15,16,...).

This w is not in 1 because

w1 = 1+12+13+··· ,

which diverges to ∞. But w is in 2and ∞ because

w2 =

1+122 +132 +··· < ∞

w∞ = sup(1, 12,13,...) = 1.

This is consistent with the observations that

• {vn}n∈N is not a Cauchy sequence in the 1-norm because no matter how large N is, the quantity

vn − vm1 =1

n +1+1

n +2+··· +1m

can be taken arbitrary large by appropriate choice of m ≥ n ≥ N.

• {vn}n∈N is a Cauchy sequence in the 2-norm because for all n,m ≥ N we have vn − vm22 < 1/N → 0 as N → ∞ (See Example 1.1.10). Since 2is a Banach space the vn hence converges in 2. Indeed it does.

• {vn}n∈N is Cauchy in the ∞-norm because for all n,m ≥ N we have vn −vm∞ < 1/N → 0 as N → ∞. Hence, by the Banach property of ∞ the vn converges in ∞.

11

The function space equivalent of 1 we naively define as L 1([a,b];R) :={f : [a,b] → R | f 1 < ∞},

where the 1-norm is now taken to be

b

f 1 =

|f (t)|dt. a

We allow a = −∞ and b = +∞. This definition of L 1([a,b];R) is not precise because it still depends on the definition of integral  ba|f (t)|dt. The Riemann integral definition is not ideal because one can construct a Cauchy sequence of Riemann integrable functions whose limit ing function is so crazy that its Riemann integral is no longer well defined. Hence the space L 1([a,b];R) would then fail to be complete. The desire of having a complete function space was so strong that it prompted mathematicians to look for alternative definitions of integra tion! In the beginning of the 20th century the issue was settled by Henri Lebesgue. He devised the Lebesgue measure and Lebesgue integration with respect to which the space L 1([a,b];R) is complete. The interested reader should follow a course on measure theory. The sym bol L is standard in the math literature and it is in honor of Lebesgue. The difference be tween Riemann- and Lebesgue integration only shows up in really weird functions, and in this course we need not worry about such functions. We simply accept that:

Theorem 1.2.6 (Lebesgue space L 1). L 1([a,b];R) and L 1([a,b];C) are Banach spaces in the 1-norm.

Built in in the definition of L 1is that its elements have a well defined 1-norm. This space contains all continuous functions but also many more, and they need not be bounded.

Example 1.2.7 (Several functions in L 1). All functions of Fig. 1.4 are elements of L 1([0, 1];R), except the last function: f9(t) = 1/t. Indeed  10f9(t) dt = log(t)10 = ∞.

Likewise, the counterpart of 2is the space of square integrable functions: Lemma 1.2.8 (Lebesgue space L 2). The space of square integrable functions L 2([a,b];R) :={f : [a,b] → R | f 2 < ∞}

is a Banach space with respect to the 2-norm defined as

b

f 2:=

|f (t)|2 dt. (1.5) a

Also here we allow a = −∞ and b = +∞. The top six functions of Fig. 1.4 are in L 2([0, 1];R). The final three functions are not L 2([0, 1];R).

Example 1.2.9 (Complete in L 2, not complete in C ). Consider the standard 2-norm of func tions (1.5), and take a = 0,b = 1. For every n > 0 the function fn : [0, 1] → R defined as

fn(t) =

n6/5t 0 ≤ t ≤ 1/n t−1/5 1/n < t ≤ 1

n1/5

1/n

12

f (t) = 1

0 1 sin(1/t)

f (t) = t

0 1

0 1

0 1

0 1 0 1 ln(t)

1

t

1/2

|t −1/2|

1/t

0 1

0 1

0 1

FIGURE 1.4: The first eight functions are in L 1([0, 1];R); function number nine is not. 13

is continuous. All fn are therefore in C ([0, 1];R) as well as in L 2([0, 1];R). As n goes to ∞ the pointwise limit of fn(t) is

f (t) =

0 t = 0, t−1/5 0 < t ≤ 1.

This function is not in C ([0, 1];R) because it is not continuous and in fact it is not bounded. It is in L 2([0, 1];R), however, because the improper integral converges:

f 22 =

1 0

f2(t) dt =

1 0

t−2/5 dt =53t3/510=53.

One can show that fn is a Cauchy sequence in the 2-norm (Exercise 1.10). Since the space L 2([a,b];R) is complete (says Lemma 1.2.8) in this norm, and since fn is a Cauchy sequence in this norm, it follows that limn→∞ fn exists in L 2([a,b];R). Indeed.

The space C ([a,b];R) is not complete in the 2-norm, and hence even though fn is a Cauchy sequence, its limit is not guaranteed to exist in the space of continuous functions C ([a,b];R). In fact, it is not hard to show that no continuous function g exists such that limn→∞ fn − g 2 = 0.

But wait, we glossed over an unsettling problem: part of the definition of norm is that f  > 0 for all f = 0,

but for L 1([a,b];R), and also for L 2([a,b];R), that is not the case! For example, function number five of Fig. 1.4 — the function in the middle — defined as

f (t) =

1 t = 1/2

0 elsewhere

is not the zero function, yet its 1-norm is zero. The simplistic way out of this problem is to identify every function f with zero norm with the zero function. That is not far fetched because if f 1 = 0 then

b

f 1 =

|f (t)|dt = 0, a

implying that f (t) is zero “almost everywhere2”. From now on we do not distinguish between functions f , g ∈ L 1 whenever their difference has norm zero, so from now on by definition we say

f = g ⇐⇒ f − g 1 = 0.

And likewise for f , g ∈ L 2.

2In a course on measure theory this identification will be formalized through equivalence classes and then the notion of almost everywhere will be properly defined.

14

1.3 Module 2 summary: inner product

What is missing in a normed vector space is the notion of “angle” and “orthogonality” be tween vectors. We will see in this section that many (but not all) normed vector spaces can be equipped with an inner product. This generalizes the dot product of Rnand, like the dot product, allows to define orthogonality. We assume familiarity with inner products on finite dimensional vector space.

Definition 1.3.1 (Real inner product). Let V be a real vector space. A function 〈·,·〉: V×V → R is a (real) inner product if for every x, y, z ∈ V and every α ∈ R the following three axioms hold,

1. 〈x, y〉 = 〈y,x〉, (symmetric) 2. 〈αx + z, y〉 = α〈x, y〉 + 〈z, y〉, (linear in first argument) 3. 〈x,x〉 > 0 for all x = 0. (positive definite)

That is all. The second axiom for x = z and α = −1 demonstrates that 〈0, y〉 = 0. This combined with the third axiom shows that 〈x,x〉 ≥ 0 for every x and that 〈x,x〉 = 0 if and only if x = 0.

Once we settle on this definition we are forced to conclude that there are inner products on Rnthat differ from the dot product. Indeed we could equip R2for instance with the fol lowing inner product:

Example 1.3.2. Consider as inner product on R2

〈x, y〉 :=x1 y1 +3x2 y2.

It is an inner product because it is well defined (in R) for every x, y ∈ R2and 1. 〈x, y〉 = x1 y1 +3x2 y2 = y1x1 +3y2x2 = 〈y,x〉,

2. 〈αx+z, y〉 = (αx1+z1)y1+3(αx2+z2)y2 = α(x1 y1+3y2x2)+(z1 y1+3z2 y2) = α〈x, y〉+〈z, y〉, 3. if x is nonzero then at least one of x1,x2 is nonzero. Then 〈x,x〉 = x21 +3x22 > 0.

As with norms, the definition of inner product implicitly says that it needs to exist for every two elements of the vector space.

Example 1.3.3. On C ([a,b];R) the integral

b

〈f , g 〉 := a

f (t)g (t) dt (1.6)

is an inner product. Let us verify (including existence):

0. If f , g ∈ C ([a,b],R) then the product f g is continuous (and bounded) on [a,b]. So then the integral 〈f , g 〉 := baf (t)g (t) dt exists and is a real number.

1. 〈f , g 〉 =  baf (t)g (t) dt = bag (t)f (t) dt = 〈g , f 〉.

2. 〈αf + g ,h〉 =  ba(αf (t)+ g (t))h(t) dt = ba αf (t)h(t) dt + bag (t)h(t) dt = α〈f ,h〉 + 〈g ,h〉.

3. If f is not the zero function then |f (t0)| > 0 for at least one t0 ∈ [a,b]. By continuity then 〈f , f 〉 =  baf2(t) dt > 0.

15

On C ([a,b];R) the inner product exists for every two of its elements. On the bigger space F([a,b];R) — the space of all functions from [a,b] to R — the integral (1.6) is not an inner product for the simple reason that the integral (1.6) is then not always convergent. Consider for instance the functions

f (t) = g (t) =

0 if t = a,

1

t−aif a < t ≤ b.a b

See also Exercise 1.14.

Also complex vector space can be equipped with an inner product. The inner product is then a function that maps to the complex numbers. We want to maintain, however, that 〈x,x〉 is a real number, in fact nonnegative:

Definition 1.3.4 (Inner product on a complex vector space). Let V be a complex vector space. A function 〈·,·〉 : V × V → C is a (complex) inner product if for every x, y, z ∈ V and every α ∈ C the following three axioms hold,

1. 〈x, y〉 = 〈y,x〉, (conjugate symmetric) 2. 〈αx + z, y〉 = α〈x, y〉 + 〈z, y〉, (linear in first argument) 3. 〈x,x〉 > 0 for all x = 0. (positive definite)

The conjugate symmetry property implies that 〈x,x〉 = 〈x,x〉, and this is another way of saying that 〈x,x〉 is real.

Example 1.3.5. On C ([a,b];C) the integral

b

〈f , g 〉 := a

f (t)g (t) dt

is an inner product (Exercise 1.15).

On complex vector space, linearity of an inner product in its first argument does not imply linearity in its second argument. Using the properties of inner product we find that

〈x,αy + z〉 = 〈αy + z,x〉 = α〈y,x〉 + 〈z,x〉 = α〈x, y〉 + 〈x, z〉.

In particular, 〈x,αy〉 = α〈x, y〉. This property of inner products is called conjugate linearity. Norm on inner product space & Cauchy-Schwarz inequality

On an inner product space (a vector space with inner product) we always define the norm via the inner product:

Definition 1.3.6 (norm associated with inner product). The norm x on a vector space with inner product is defined as x = 〈x,x〉.

It is also possible to define norm without having inner product (see Definition 1.1.1); all that the above says is that if we have an inner product space then we always define the norm accordingly. Of course this assumes that the above satisfies the defining properties of norm. It does:

16

Lemma 1.3.7 (Norm). Let V be a real or complex vector space with inner product. The norm ·  associated with the inner product exists and satisfies the axioms of norm:

1. αx = |α|x, (positive homogeneous) 2. x + y ≤ x + y, (triangle inequality) 3. x > 0 for all x = 0. (positive definite)

Proof. x exists because 〈x,x〉 exists. The first axiom follows from αx =

〈αx,αx〉 =

αα〈x,x〉 = |α|x. The third axiom is obvious. The second (the triangle inequality) is more

involved. It follows from Cauchy-Schwarz’ inequality, which we will prove shortly (§ 1.3).

Since an inner product induces a norm, it is natural to ask whether every norm can be seen as induced by an inner product. That is not the case. In fact one can elegantly charac terize which norms have an associated inner product.

Lemma 1.3.8 (Parallelogram law). Let  ·  be a norm on a real or complex vector space X. There is an inner product on X such that x = 〈x,x〉 for all x ∈ X iff the norm satisfies the parallelogram law

2(x2 + y2) = x − y2 + x + y2,

see Fig. 1.5. In that case the inner product follows uniquely from the norm as 〈x, y〉 =x + y2 − x − y2

4(1.7) on real vector space, or as

4+ ix + iy2 − x − iy2

〈x, y〉 =x + y2 − x − y2

on complex vector space. Proof. See Exercise 1.16.

4(1.8) x

yx + y x + y

y

x − y

y

0x

x

FIGURE 1.5: The parallelogram law on R2 with norm x = x21 + x22says that the sum of squares of the lengths of the four outer sides of the parallelogram, 2(x2 + y2), equals the sum of squares of the lengths of the two diagonals, x − y2 + x + y2.

Example 1.3.9. Consider the three norms on R2as used in Example 1.1.2:

x1 = |x1| +|x2|,

x2 =

x21 + x22,

x∞ = max(|x1|,|x2|).

17

The 1-norm has no associated inner product because for x =10and y =01the parallelo gram law fails:

2(x21 + y21)

=4

= x − y21 + x + y21

.

=8

The 2-norm does have an inner product because x2 equals 〈x,x〉 for 〈x, y〉 :=x1 y1 + x2 y2. (For completeness we should show that the above is indeed an inner product. Do this yourself. Alternatively we could verify the parallelogram law for the 2-norm.)

The ∞-norm has no associated inner product because for x =10and y =01the paral lelogram law fails:

2(x2∞ + y2∞)

=4

= x − y2∞ + x + y2∞

=2

.

Orthogonal complement

We use x ⊥ y to mean that 〈x, y〉 = 0, so orthogonality is inner product dependent. Like wise for any nonempty subset S of some inner product space X we use S⊥ to mean the orthogonal complement of S, i.e. the set of all vectors in X that are orthogonal to all elements of S,

S⊥ = {x ∈ X | 〈x,s〉 = 0∀s ∈ S}.

Example 1.3.10. Consider again the non-standard inner product of Example 1.3.2 on R2, 〈x, y〉 :=x1 y1 +3x2 y2.

In this norm the so-called unit!ball {x ∈ R2| x ≤ 1} is not a disc but an ellipse {x ∈ R2| x ≤ 1} = {x ∈ R2| x21 +3x22 ≤ 1},

see Fig. 1.6(a). The orthogonal complement of the set S consisting of the single element,

S :=

is now

1

−1

S⊥ =

1

−1

⊥

= {x | 〈1−1,x〉 = 0} = {x | x1 −3x2 = 0} = span{31},

see Fig. 1.6(b). Orthogonality with respect to our inner product here does not mean “having an angle of π/2”.  The Pythagorean theorem remains valid on arbitrary inner product space: Lemma 1.3.11 (Pythagoras). If x ⊥ y then x + y2 = x2 + y2.

Proof. If 〈x, y〉 = 0 then

x + y2 = 〈x + y,x + y〉

= 〈x,x + y〉 + 〈y,x + y〉

= 〈x,x〉 + 〈x, y〉

0

+ 〈y,x〉

0

+〈y, y〉 = x2 + y2.

We used here that 〈y,x〉 = 〈x, y〉 = 0.

18

x ≤ 10 S

span [ 31] span [ 1−1]

FIGURE 1.6: (a) Unit ball for inner product 〈x, y〉 = x1 y1 +3x2 y2; (b) two orthogonal vec tors in this inner product.

In fact on real vector space, x ⊥ y is equivalent to x + y2 = x2 + y2, but on complex vector space it is not equivalent, see Exercise 1.24.

Example 1.3.12 (Orthogonal sinusoids). Consider C ([0, 2π];R) with inner product 〈f , g 〉 =  2π

0f (t)g (t) dt. On this space sine and cosine are orthogonal,

sin ⊥ cos.

Why? Because

〈sin,cos〉 =

2π 0

cos(t)sin(t) dt =

2π 0

2sin(2t) dt =−cos(2t)

1

4

2π 0

= 0.

See Fig. 1.7(botttom left) for a “proof by picture” of orthogonality of these two functions. The Pythagorean theorem now implies that sin+cos2 = cos2 + sin2 = π + π = 2π, see Fig. 1.7(right).

2π

0

cos(t)

sin(t)

sin2(t) cos2(t)

1

0

(sin(t)+cos(t))2

2π

0

sin(t)cos(t)

2π

1 0

2π

FIGURE 1.7: The graph shown in the bottom-left figure suggests that sin(t)cos(t) is zero on average over [0, 2π]. Hence 〈sin,cos〉 = 0. The top-right figure shows that sin2 = cos2 = π, and then the bottom-right figure shows that sin+cos2 = 2π.

Cauchy-Schwarz inequality

Let x and y be elements of some inner product space. Fig. 1.8 suggests that x can uniquely be written as a sum of two vectors where one is in span{y} and the other is orthogonal to this span, i.e. that

x = αy + y⊥

19

x

y

y⊥αy

0

FIGURE 1.8: x = αy + y⊥.

for some unique αy and unique vector y⊥ orthogonal to y. That is indeed the case because either y = 0 in which case we have αy = 0 and y⊥ = x, or else

x = αy + y⊥ ⇐⇒ (x −αy) ⊥ y

⇐⇒ 〈x −αy, y〉 = 0

⇐⇒ 〈x, y〉 −α〈y, y〉 = 0

⇐⇒ α =〈x, y〉

y2.

Hence α is unique, and therefore αy and y⊥ :=x −αy are unique as well. Pythagoras says that then

x2 = αy2 + y⊥2 =|〈x, y〉|2

y2+ y⊥2. (1.9)

We derived here a one-dimensional version of the projection theorem (discussed later) but more importantly we derived that inner products can be bounded by norms. This is the fun damental inequality of Cauchy-Schwarz:

Theorem 1.3.13 (Cauchy-Schwarz inequality). Every inner product satisfies |〈x, y〉| ≤ xy ∀x, y,

and equality holds iff x and y are linearly dependent, i.e. iff x = αy or y = αx for some scalar α.

Proof. The result is trivial if y = 0. For y = 0, Inequality (1.9) says that

x2y2 = |〈x, y〉|2 + y⊥2y2 ≥ |〈x, y〉|2,

and so the result follows. We have equality iff y⊥ = 0 and that is the case iff x and y are linearly dependent.

Example 1.3.14 (Aligned vectors in R4). Let

x =



101010 5



, y =



111 β



.

Then in the standard inner product (dot product) on R4 we have

1

xy=30+5β

〈x, y〉

300+253+β2.0 β → 1/2

20

The two vectors are linearly dependent (aligned) iff β = 1/2. Hence

〈x, y〉

xy= ±1 iff β = 1/2.

The fact that the ratio is plus 1 at β = 1/2 means that x and y are positively aligned, i.e. that x = αy for some positive number α.

The Cauchy-Schwarz inequality is very important and shows up in numerous applica tions, such as signal detection problems because we can use 〈x,y〉

xyas a practical measure of

“similarity” of x and y. A consequence of Cauchy-Schwarz is the triangle inequality. Lemma 1.3.15 (Triangle inequality). For any inner product and associated norm we have x + y ≤ x + y.

Proof.

x + y2 = 〈x + y,x + y〉

= 〈x,x〉 + 〈x, y〉 + 〈y,x〉 + 〈y, y〉

= 〈x,x〉 + 〈x, y〉 +〈x, y〉 + 〈y, y〉

= 〈x,x〉 +2Re(〈x, y〉)+ 〈y, y〉

≤ 〈x,x〉 +2|〈x, y〉| + 〈y, y〉

≤ x2 +2xy + y2

= (x + y)2.

1.4 Orthogonal projection

x

V v∗

FIGURE 1.9: The best approximation v∗ ∈ V of x has the property that x−v∗ is orthogonal to the subspace V.

With inner products comes along a notion of orthogonality and a notion of distance (norm), so we can talk about “orthogonal projections” and “best approximations”.

Definition 1.4.1 (Best approximation). An element v∗ ∈ V is said to be a best approximation in V of x if

x − v∗ ≤ x − v ∀v ∈ V.

21

Figure 1.9 suggests that best approximations v∗ in a subspace V are such that x − v∗ is orthogonal to V. That is correct, in fact, the orthogonality property is both necessary and sufficient for optimality:

Theorem 1.4.2 (A projection theorem). Let X be a vector space with inner product, V a sub space of X, and x an element of X. Then

1. a v∗ ∈ V is a best approximation in V of x iff (x − v∗) ⊥ V,

2. if the best approximation v∗ ∈ V exists then it is unique and it satisfies x − v∗2 = x2 − v∗2.

Proof. Suppose x −v∗ ⊥ V for some v∗ ∈ V. Then for any v ∈ V the difference v −v∗ is in V by the subspace property, and so by Pythagoras we get

x − v2 = (x − v∗)

∈V⊥

−(v − v∗)

∈V

2 = x − v∗2 + v − v∗2 ≥ x − v∗2.

Hence if v = v∗ then the norm of x − v exceeds that of x − v∗, making v∗ the unique best approximation.

Conversely, suppose x − v∗ ⊥ V. Then by definition there is a v ∈ V such that x − v∗ ⊥ v i.e. such that 〈x − v∗, v〉 = 0. In particular this v is nonzero. We construct an improved approximation of x of the form v∗ +αv with the scalar α yet to be determined.

x −(v∗ +αv)2 = (x − v∗)−αv2

= x − v∗2 −2Re〈x − v∗,αv〉 + αv2

= x − v∗2 −2Re(α〈x − v∗, v〉)+|α|2v2.

For α = 〈x − v∗, v〉/v2this becomes

= x − v∗2 −2|〈x − v∗, v〉|2

v2+|〈x − v∗, v〉|2

v2

= x − v∗2 −|〈x − v∗, v〉|2

v2< x − v∗2.

This shows that v∗ is not a best approximation.

The equality x − v∗2 = x2 − v∗2is a restatement of Pythagoras, see Fig. 1.9.

With this theorem we can in many cases calculate best approximations. More on this soon, but first a puzzling example. It may happen that a best approximation does not exist:

Example 1.4.3 (No solution exists). Consider the inner product space X = 2and its sub space finite of finitely nonzero sequences. There is no best approximation v∗ ∈ finite of the infinite sequence

x = (1, 12,13,14,15,...)

because it can be approximated arbitrarily well by vn :=(1, 12,13,...,1n, 0, 0,...) but the approxi mation error cannot be zero because x ∈ finite.

This example demonstrates that best approximations need not exist. For finite dimen sional subspaces, however, best approximations always do exist, and in fact can be deter mined explicitly:

22

Lemma 1.4.4 (Projection onto finite dimensional subspace). Consider a real or complex inner product space X, and subspace V ⊆ X, and assume that V is spanned by a finite set {v1,..., vn}.

1. Then v∗ is a best approximation in V of x if-and-only-if v∗ =ni=1αi vifor some α ∈ Rn (or Cn) that satisfies





〈v1, v1〉 〈v2, v1〉 ··· 〈vn, v1〉 

〈v1, v2〉 〈v2, v2〉 ··· 〈vn, v2〉 ............

〈v1, vn〉 〈v2, vn〉 ··· 〈vn, vn〉

α =



〈x, v1〉

〈x, v2〉 ...

〈x, vn〉



. (1.10)

Gram matrix G

The so defined Gram matrix G is square.

2. If {v1,..., vn} is a basis of V then the Gram matrix is invertible, and for every x ∈ X the solution α exists and is unique, and the best approximation v∗ exists (and is unique).

3. If {v1,..., vn} is not a basis of V then the Gram matrix is not invertible, but for every x ∈ X a solution α of (1.10) still exists (though not unique), and the best approximation v∗ exists (and is unique).

Proof. Write the candidate best approximation v∗ as v∗ =ni=1αi vi.

1. By the projection theorem, a vector v∗ ∈ V is a best approximation of x iff n

(x − v∗) ⊥ V ⇐⇒ x − ⇐⇒ x −

αi vi⊥ span{v1,..., vn}

i=1

n

αi vi⊥ vk ∀k = 1,...,n i=1

n

⇐⇒ 〈x, vk 〉 − i=1

αi〈vi, vk 〉 = 0 ∀k = 1,...,n

⇐⇒ 〈x, vk 〉 −〈v1, vk 〉 〈v2, vk 〉 ··· 〈vn, vk 〉α = 0 ∀k = 1,...,n

⇐⇒ Equation (1.10) holds.

2. We show that the Gram matrix is invertible if {v1,..., vn} is a linearly independent set. We use that the Gram matrix does not depend on x. Consider now the best approxi mation v∗ of x = 0. Clearly this is v∗ = 0 and it is unique. For x = 0, Equation (1.10) becomes Gα = 0 where G is the Gram matrix. If G would have been singular then more than one solution α would have existed, implying (by linear independence) that more than one best approximation v∗ =ni=1αi vi would have existed. That is not the case, i.e. G is not singular if {v1,..., vn} is independent.

3. If {v1,..., vn} is not linearly independent then, for some nonzero α, we have that v∗ :=ni=1αi viis zero. Clearly this is the best approximation of x = 0, so this α must satisfy (1.10), i.e. Gα = 0. Hence G is not invertible. Now either V = {0}, in which case all vi are zero, and then the result trivially holds. Or V = {0}. In that case V has a basis, so by part 2, a best approximation v∗ does exist. If we write this v∗ as v∗ =ni=1αi vi, then, by part 1, this α satisfies (1.10).

23

Equation (1.10) is also known as the normal equation(s). For n = 1 the normal equations state that

v∗ = αv1, α =〈x, v1〉

〈v1, v1〉.

This recovers the one-dimensional projection formula that we used to prove the Cauchy Schwarz inequality. Now with the Gram matrix we can determine orthogonal projections on subspaces of finite dimension:

Example 1.4.5. Let us redo the projection of Example 1.3.10, that is, we want find the best approximation in span{v1} for v1 =1−1of

x =

2

2

with the non-standard inner product 〈x, y〉 = x1 y1+3x2 y2. According to the normal equations we have as best approximation

v∗ = v1〈x, v1〉

〈v1, v1〉=

1

−1

2×1+3×2×(−1)

1+3=

1

−1

−4

4=

−1 1

.

See Fig. 1.10.  x

v∗

0

V⊥ V

FIGURE 1.10: The best approximation in span (1,−1) of x = (2, 2) with respect to inner product 〈x, y〉 = x1 y1 +3x2 y2 is v∗ = (−1, 1). See Example 1.4.5.

Example 1.4.6. Consider C ([−1, 1];R) with standard inner product 〈f , g 〉 =  1−1f (t)g (t) dt. We determine the best approximation of the function etin the subspace of polynomials of degree at most 1. That is, we need to solve

α1,α2∈Ret −(α1 +α2t).

min

A basis of this subspace of polynomials is {1,t}, and with this choice the normal equa tions (1.10) become

〈1, 1〉 〈t, 1〉

〈1,t〉 〈t,t〉

α1 α2

=

〈et, 1〉 〈et,t〉

.

The six inner products here equate to

2 0

0 2/3

α1 α2

=

e− e−1 2e−1

.

24

Its solution is  1

α1

α2

=

2(e− e−1) 3e−1

.

The best approximation on [−1, 1] of et by a degree one (or less) polynomial thus is p∗(t) =12(e− e−1)+3e−1t,

see Fig. 1.11.

et

p∗(t)

−1 1

FIGURE 1.11: Best approximation of et by degree-1 polynomial p∗(t). This polynomial minimizes the norm of the approximation error et − p∗(t) (shown in gray). See Exam ple 1.4.6.

Orthonormal sequence and Parseval identity

A drawback of the Gram matrix is that it does not easily extend to infinite dimensional sub spaces (its Gram matrix would be an infinite matrix). To avoid such problems it is customary to assume more structure, namely the case where the Gram matrix (1.10) is the identity ma trix:

Definition 1.4.7 (Orthonormal sequence). A sequence {e1,e2,e3,...} in some inner product space, is an orthonormal sequence if all elements have unit norm and are mutually orthogo nal: ek  = 1 ∀k and ek ⊥ e j ∀k = j.

Verify for yourself that the Gram matrix (1.10) is the identity matrix if {v1,..., vn} is an orthonormal sequence {e1,...,en}. As a result we immediately get that the solution α of (1.10) then is

αi = 〈x,ei〉,

and, therefore, that the best approximation of x in span{e1,...,en} is

v∗ = 〈x,e1〉e1 + 〈x,e2〉e2 +··· + 〈x,en〉en. (1.11)

In the case of an orthonormal sequence, the coordinates αi:=〈x,ei〉 are referred to as Fourier coefficients. The orthonormal property also gives us that

v∗2 =n k=1

|〈x,ek 〉|2.

25

If x happens to be an element of span{e1,...,en} then, obviously, v∗ = x so then

x2 =n k=1

|〈x,ek 〉|2. (1.12)

This is known as Parseval’s identity and it should remind you of the fact that in Rnthe Eu clidean norm of the vector can be expressed by its coordinates with respect to the standard (orthonormal) basis as x = (x21 + x22 +··· + x2n)12 . Now an important example.

Example 1.4.8 (Finite real Fourier series). Consider C ([−1, 1];R) with standard inner prod uct,

1

〈f , g 〉 =

−1

f (t)g (t) dt.

Similar to Example 1.3.12 we claim that cos(πt) and sin(πt) are orthogonal. In fact all cosines and sines with period π are orthogonal! We mean that for whatever n, the set of 2n + 1 har monic functions

{1/2, cos(πt), sin(πt), cos(2πt), sin(2πt),..., cos(nπt), sin(nπt)}

is an orthonormal set (Exercise 1.36). In particular the set is therefore independent. To keep matters simple, let us determine the best approximation of some f ∈ C ([−1, 1];R) in the span of only 1, cos(πt), sin(πt). That is we want to determine

f∗(t)=c0+c1 cos(πt)+c2 sin(πt)f − f∗.

min

Exploiting the orthonormality of {e1,e2,e3} :={1/2,cos(πt),sin(πt)} we find

f∗(t) = 〈f ,e1〉e1(t)+ 〈f ,e2〉e2(t)+ 〈f ,e3〉e3(t)

= 〈f ,12〉12+ 〈f ,cos(π·)〉cos(πt)+ 〈f ,sin(π·)〉sin(πt).

For f (t) = t2the best approximation turns out to be f∗(t) =13−4π2 cos(πt), see Fig. 1.12.  f (t) = t2

f∗(t) =13−4π2 cos(πt)

t →

−1 0 1

FIGURE 1.12: Function f (t) = t2 on [−1, 1] and its best approximation f∗(t) in span{1/2,cos(πt),sin(πt)}. The error f (t)− f∗(t) is shown in gray.

26

Example 1.4.9 (Legendre polynomials). Without proof we claim that the sequence of poly nomials

p0(t) = 1,

p1(t) = t,

pk+1(t) =(2k +1)t pk (t)−kpk−1(t)

k +1∀k ≥ 1

defines an orthogonal sequence of polynomials on C ([−1, 1];R) with the standard inner prod uct 〈f , g 〉 =  1−1f (t)g (t) dt. Their norms are

pk  = 2

2k +1.

See Fig. 1.13. So then ek:=

2k+1

2pk is an orthonormal sequence. The pk are known as the

Legendre polynomials in honor of Adrien-Marie Legendre (1752-1833) who introduced them to describe Newtonian potentials. Legendre polynomials also play a role in numerical analysis (google for “Gaussian quadrature” if you want to know).

p0 p1 p2 p3

p4 p5 p6 p7 FIGURE 1.13: The first eight Legendre polynomials pk : [−1, 1] → R.

1.5 Hilbert space

Inner product spaces may be incomplete, and this is unfortunate since then all sorts of limits are not guaranteed to exist. For instance best approximations et cetera need not exist (Exam ple 1.4.3). In this final section we focus on inner product spaces that are complete. These are known as Hilbert spaces in honor of the German mathematician David Hilbert (1862–1943). Hilbert spaces by definition enjoy all the properties of Banach spaces and in addition they have the rich structure brought about by the inner product. Most of our geometrical intuition for R2and R3generalizes to arbitrary Hilbert space.

Definition 1.5.1 (Hilbert space). A Hilbert space is an inner product space that is complete in the norm x :=〈x,x〉.

So a Hilbert space is a Banach space whose norm satisfies the parallelogram law. This law is usually easy to check. Every inner product space that is finite dimensional is a Hilbert space. For example

27

• Rnin whatever inner product, for instance the dot product, is a Hilbert space;

• The set of polynomials from [a,b] to R of degree n or less, with the standard inner prod uct 〈f , g 〉 =  baf (t)g (t) dt, is a Hilbert space;

• Cnin whatever inner product, for instance 〈x, y〉 = nk=1xk yk, is a Hilbert space. Arguably the most interesting Hilbert space for our course is 2:

Example 1.5.2 (Square-summable sequence space). Theorem 1.2.4 claims that the space of real, square summable sequences

2 = {(u1,u2,...) | uk ∈ R,u2 < ∞},

is a Banach space in the standard 2-norm u2:=∞k=1u2k. This norm corresponds to the inner product defined as

∞

〈v,w〉2 =

k=1

vkwk .

Hence 2in this inner product is a Hilbert space.

Similarly, 2(N;C) = {(u1,u2,...) | uk ∈ C,∞k=1|uk |2 < ∞} is a Hilbert space in the complex inner product 〈x, y〉2 =∞k=1ykxk . The spaces 1and ∞ are not Hilbert spaces because they are not even inner product spaces (Exercise 1.43). The Lebesgue space equivalent of 2is L 2 and this, again, is a Hilbert space:

Lemma 1.5.3 (Square integrable function space). Let a,b ∈ R, a < b, possibly a = −∞ and/or b = ∞. The set

L 2([a,b];R) :={f : [a,b] → R |

b a

f2(t) dt < ∞}

is a Hilbert space in the standard inner product  b

〈f , g 〉 =

a

g (t)f (t) dt. (1.13)

From one Hilbert space one can generate others.

Theorem 1.5.4 (Orthogonal complement). Let S be a non-empty subset of a Hilbert space X. Then the orthogonal complement S⊥ is a Hilbert space.

Proof. First of all, S⊥ is a subspace and therefore a vector space. Remains to show that every Cauchy sequence in S⊥ converges in S⊥. Let fn be a Cauchy sequence in S⊥. Since X is Hilbert, the sequence fn has a limit f in X. Now by continuity of inner product (Exercise 1.52) we have for every s ∈ S that 〈f ,s〉 = limn→∞〈fn,s〉 = 0. So f ∈ S⊥.

Example 1.5.5 (Even functions). The orthogonal complement of the set S of odd functions S = {f ∈ L 2(R;R) | f (t) = −f (−t) ∀t}

are the even functions (verify this). So the set of even functions in L 2(R;R) is a Hilbert space.

28

The projection theorem 1.4.2 claims that if a best approximation exists then it is unique. It does not say that a best approximation exists and in fact it need not exist (Example 1.4.3). In that example, we projected onto a space that is not a Hilbert space. Could it be? Yes:

Theorem 1.5.6 (Projection theorem, final version). Let X be an inner product space and suppose V ⊆ X is a Hilbert space. Then every x ∈ X has a best approximation v∗ ∈ V, and the best approximation is unique.

Proof. Uniqueness we showed earlier (Thm. 1.4.2). Existence we show now. Let β = infv∈V x − v. This implies that a sequence vn ∈ V exists such that

x − vn2 ≤ β2 +1n∀n ∈ N.

Below we show that vn is Cauchy. Then by completeness of Hilbert space we have that v∗ :=limn→∞ vn is in V. Now by continuity of norm we have x − v∗ = β, making v∗ a best approximation. Done.

Proof that vn is Cauchy. By the parallelogram law we have

vn − vm2 + vn + vm −2x2 = 2vn − x2 +2vm − x2,

see Fig. 1.14. Therefore

vn − vm2 = 2vn − x2 +2vm − x2 − vn + vm −2x2

2.

Since vn+vm

= 2vn − x2 +2vm − x2 −4x −vn+vm 2

2∈ V and since β is the smallest possible error norm, we have that x −vn+vm

Therefore

vn − vm2 ≤ 2vn − x2 +2vm − x2 −4β2

≤ 2(β2 +1n)+2(β2 +1m)−4β2 = 2( 1n+1m).

It follows that vn is a Cauchy sequence.

vn + vm −2x

2 ≥ β.

vm − x

x

−

vm

vn − x

vn + vm −2x vn − vm

0vn − x

FIGURE 1.14: Parallelogram law.

Complete orthonormal system

Many familiar results from Rncarry over to arbitrary Hilbert space. For instance the following central and truly powerful theorem. It combines many results. For instance it generalizes the notion of basis to infinite dimensional Hilbert space.

29

Theorem 1.5.7 (Complete orthonormal system). Let X be a Hilbert space and let {e1,e2,...} be an orthonormal sequence (possibly infinite) in X. The following conditions are equivalent.

1. x ⊥ ek for all ek iff x = 0, (sequence is complete) 2. x =k〈x,ek 〉ek ∀x ∈ X, (x equals Fourier series) 3. x2 =k|〈x,ek 〉|2 ∀x ∈ X. (Parseval’s identity)

In that case we say that {e1,e2,...} is a orthonormal basis or a complete orthonormal system of X.

Proof. We prove it for infinite sequences {e1,e2,...} (the finite case is simpler). Proof of (1) =⇒ (2): for any n, the finite sum xn defined as

n

xn =

k=1

〈x,ek 〉ek

is the best approximation of x in span{e1,...,en} hence by Thm. 1.4.2 the norm of the approxi mation xn does not exceed that of x, so nk=1〈x,ek 〉ek  ≤ x < ∞. By Pythagoras this means that the sequence of positive real numbers,

αn :=

n

k=1

|〈x,ek 〉|2

are bounded from above by x2 < ∞. So αn is an increasing but bounded sequence. By the monotone convergence theorem it therefore converges, α∞ = limn→∞ αn. But then xn is a Cauchy sequence because for any m ≥ n ≥ N,

xn − xm2 = n<k≤m

|〈x,ek 〉|2 = αm −αn ≤ α∞ −αN

which converges to 0 as N → ∞. By the Hilbert property then x∞ :=∞k=1〈x,ek 〉ek exists in X. This x∞ equals x by the following argument: for every index k there holds

〈x − x∞,ek 〉 = 〈x,ek 〉 − 〈 limn→∞xn,ek 〉

= 〈x,ek 〉 − limn→∞〈xn,ek 〉 by continuity of inner product

n

= 〈x,ek 〉 − limn→∞〈

〈x,em〉em,ek 〉 m=1

n

= 〈x,ek 〉 − limn→∞

〈x,em〉〈em,ek 〉 m=1

= 〈x,ek 〉 − 〈x,ek 〉 = 0 ∀k.

By completeness of the {e1,e2,...} this implies that x − x∞ = 0, i.e. that x = x∞. Proof of (2) =⇒ (3): this is immediate from Pythagoras, continuity of norm and orthonor mality of {e1,e2,...}:

x2 =  limn→∞xn2 = limn→∞xn2 = limn→∞n k=1

|〈x,ek 〉|2.

Proof (3) =⇒ (1): if x ⊥ ek for all k then x2 =k|〈x,ek 〉|2 = 0 hence by the positive definite property of norm we have x = 0.

30

For finite dimensional space such as Rnthe theorem is intuitive and says that {e1,e2,...} span the entire space iff the orthogonal complement of span{e1,e2,...} is zero.

Lemma 1.5.8 (A polynomial orthonormal basis of L 2). Consider the Hilbert space L 2([−1, 1];R) with standard inner product 〈f , g 〉 =  1−1f (t)g (t) dt. There is an orthonor mal sequence of polynomials {ek }k=0,1,2,... with ek having degree k. Moreover this sequence is then an orthonormal basis of L 2([−1, 1];R).

Proof. Let vk−1(t) be the best approximation of tkin the space of polynomials of degree at most k−1. Then ek:=(tk−vk−1)/tk−vk−1 is an orthonormal sequence with ek having degree k. (See also Example 1.4.9.)

According to Thm. 1.5.7 it suffices to show that the only function f ∈ L 2([−1, 1];R) or thogonal to all ek is the zero function. So assume that 〈f ,ek 〉 = 0 for all k = 0, 1,..., i.e.

1 −1

f (t)tkdt = 0, k = 0, 1, 2,....

Assume, for the moment, that f is continuous. If f is nonzero then f is > 0 (or < 0) through out some small enough interval [c,d], see Fig. 1.15. Next let h(t) be a parabola that exceeds 1

h(t)

1

f (t)

−1 1 c d

FIGURE 1.15: A parabola h(t) so that h(t) ≥ 1 on [c,d] and |h(t)| < 1 elsewhere on [0, 1].

if c < t < d, and for all other t < c and t > d is positive but less than 1. Such a parabola exists. Realize that every power hn of h is a polynomial (in fact hn has degree 2n so is an element of span{e0,e1,...,e2n}. The higher the power, the larger hnis in the interval [c,d] and the smaller it is elsewhere. Therefore for some large enough n we have

〈f ,hn〉 = 0.

This means that f can not be orthogonal to all ek . Hence the only continuous f that is or thogonal to all ek is the zero function, f = 0.

Now let f ∈ L 2([0, 1];R), not necessarily continuous, and assume that f ⊥ ek for all k = 0, 1,.... Again we show that f is then necessarily the zero function. Consider its antiderivative

t

F(t) :=

−1

f (τ) dτ.

This function is continuous. It is also orthogonal to all ek because

〈F,ek 〉 =

1 −1

F(t)tkdt =

F(t)tk+1 k +1

1

−1

−

1 −1

f (t)tk+1

k +1dt = 0, ∀k = 0, 1, 2,....

Here we used that F(0) = 0 and F(1) = 1−1f (t) dt = 〈f , 1〉 = 0. So F is orthogonal to ek for all k = 0, 1,.... Since F is continuous and orthogonal to all ek we have that F is the zero function. Its derivative f hence is zero as well, which is what we set out to prove.

31

So every square integrable function f : [−1, 1] → R can be approximated (in the 2-norm) arbitrary well by polynomials ek , and, in fact, f =∞k=0fk ek for fk = 〈f ,ek 〉. Not bad. In Chap ter 3 we will see that not only polynomials have this property but also harmonic functions (see Theorem 3.1.1).

1.6 Exercises

1.1 Consider the three norms on R2as defined in Example 1.1.2.

(a) Show that the 1-norm is a norm.

(b) Show that the 2-norm is a norm. (The triangle inequality is a tricky one to verify; you probably want to read § 1.3 first.)

(c) Show that the ∞-norm is a norm.

1.2 Prove the claim of Example 1.1.5.

1.3 Prove the claim of Example 1.1.7.

1.4 Suppose · a and · b are two norms on the same vector space. Is · a + · b a norm? 1.5 Show that ·  is a norm on R if and only if

x = γ|x|

for some real number γ > 0. [Hint: use that a vector x ∈ R can also be seen as the scalar x ∈ R times the vector 1 ∈ R.]

1.6 Suppose X is a finite dimensional vector space with basis V = {v1,..., vn}. Denote the coordinate vector of x ∈ X as xV = (xV,1,...,xV,n). Is

n

x :=

k=1

|xV,k |

a norm on X?

1.7 Let XA and XB be two real vector spaces, and suppose  · A is a norm on XA and  · B a norm on XB . Show that

(xa,xb) :=max(xaA,xbB )

is a norm on the cartesian product space

XA ×XB :={(xa,xb) | xa ∈ XA,xb ∈ XB }

1.8 Let P2(R;R) be the set of polynomials p : R → R that have degree 2 or less. It is a sub space of F(R;R). Let p0,p1,p2 ∈ R. Is

p0 + p1t + p2t2 :=|p0| +|p1| +|p2|

a norm on P2(R;R)?

1.9 Let {xn}n∈N be a Cauchy sequence in some normed vector space. Show that the se quence of norms an := xn is a Cauchy sequence of real numbers (in the vector space R with absolute values as its norm).

32

1.10 Show that the fn of Example 1.2.9 is a Cauchy sequence in the 2-norm.

1.11 Show that every Cauchy sequence is bounded (meaning: for every Cauchy sequence {fn}n∈N there is an M > 0 such that fn ≤ M for all n ∈ N).

1.12 Bounded variation. Functions of bounded variation roughly are those functions whose graph can be plotted up to any degree of precision, see Fig. 1.16. Formally, f ∈ F([a,b];R) is of bounded variation if there is an M > 0 (that may depend on f ) such that

N

k=1

|f (xk )− f (xk−1)| ≤ M

for every N and every partitioning a = x0 < x1 < ··· < xN = b of [a,b]. The total variation of f is defined as

N

V (f ) := sup

a=x0<x1<···<xN =b

k=1

|f (xk )− f (xk−1)|.

Here the supremum is taken over all N ∈ N and all ordered xk (with x0 = a,xN = b). Let BV ([a,b];R) be the subset of F([a,b];R) of functions whose total variation is finite.

(a) Argue that V (f ) = f (b)− f (a) if f is a non-decreasing function, such as f (b)

f (a)

a b

(b) Determine V (f ) for f (t) = (b − t)(t − a). A plausible argument suffices,

a b

(c) Show that BV ([a,b];R) is a subspace

(d) Show that V (f ) is not a norm on BV ([a,b];R)

(e) Show that |f (a)| +V (f ) is a norm on BV ([a,b];R)

(f ) Find a continuous f ∈ F([0, 1];R) whose total variation is infinite.

1.13 Uniqueness of limit. Let {fn}n∈N be a sequence in some normed vector space X.

(a) Suppose fn converges. Show that limn→∞ fn is unique.

(b) Let Y be a subspace of X (with the norm as on X). Suppose {fn}n∈N ⊆ Y is a se quence in Y (and hence in X) and that it converges in X to some f ∈ X. Show that {fn}n∈N converges in Y if and only if f ∈ Y. [Hint: this is an easy problem.]

(c) Use the previous part to show that the fn of Example 1.1.8 have a limit in L 1([−1, 1];R) in the 1-norm, but not in the subspace C ([−1, 1];R) in the 1-norm.

1.14 Consider Example 1.3.3. Show that for the function space F([a,b];R) also the third axiom of inner product does not hold.

33

FIGURE 1.16: Graph of t cos(1/t) on [−0.1, 0.1].

1.15 Prove the claim of Example 1.3.5

1.16 Parallelogram law. Let X be a vector space with inner product, and assume x, y ∈ X. (a) Prove that every inner product satisfies the parallelogram law:

x + y2 + x − y2 = 2x2 +2y2.

(b) Show that on real vector space we have (1.7).

(c) Show that on complex vector space we have (1.8).

1.17 Consider C ([0, 1];R) with the max-norm, f ∞ = maxx∈[0,1] |f (x)|. Let h(x) = 1, g (x) = x.

(a) Compute h∞, g ∞, h + g ∞, h − g ∞

(b) Is there an inner product whose norm is the max-norm?

1.18 Let P2([0, 1];C) be the set of polynomials p : [0, 1] → C of degree 2 or less. (a) Is

〈p,q〉 :=p(0)q(0)+ p(1)(0)q(1)(0)

an inner product on P2([0, 1];C)?

(b) Show that

〈p,q〉 :=p(0)q(0)+ p(1)(0)q(1)(0)+ p(2)(0)q(2)(0) (1.14)

is an inner product on P2([0, 1];C).

(c) Determine 〈1, ix〉 for the inner product of (1.14)

(d) What is the distance between 1 + x + x2and 2 − 2x in the inner product (1.14)? (The distance between two vectors f1 and f2 is defined as the norm f1 − f2 of their difference.)

(e) Repeat 1.18c and 1.18d for the inner product

1

〈p,q〉 = 0

p(x)q(x) dx.

34

1.19 Prove the following properties of complex inner products.

(a) 〈u1 +u2 +u3, v〉 = 〈u1, v〉 + 〈u2, v〉 + 〈u3, v〉

(b) 〈u,αv〉 = α〈u, v〉

(c) 〈u, v +w〉 = 〈u, v〉 + 〈u,w〉

1.20 Show that 〈x, y〉 = yTP x is an inner product on Rnif P = QTQ for some nonsingular n×n matrix Q.

1.21 Suppose 〈x, y〉 = 0 for all x ∈ X. Show that y = 0.

1.22 Is 〈A,B〉 :=tr(BTA) an inner product on Rn×k? Here tr is the trace, defined as the sum of the diagonal entries.

1.23 Is it S⊥⊥ ⊆ S or S ⊆ S⊥⊥?

1.24 Show that on real vector space x+y2 = x2+y2iff x ⊥ y and find a counter example for this equivalence for the complex vector space C.

1.25 Show that a c > 0 exists such that

π

0

f (t)cos(t) dt

≤ c

π 0

|f (t)|2 dt ∀f ∈ C ([0,π];R),

and determine the smallest c∗ possible and a non-zero function f (t) that achieves equality for this smallest c∗.

1.26 Consider the vector space of continuous functions f defined on the complex unit circle, f : {z ∈ C | |z| = 1} → C.

Show that

〈f , g 〉 =

2π 0

f (eiω)g (eiω) dω

is an inner product. 1.27 Consider the set

2(N;C) = {u ∈ (N;C) |∞

n=1

|un|2 < ∞}.

Its elements are the infinite sequences (u1,u2,...) whose entries un ∈ C converge to zero so fast that u2:=∞n=1|un|2is convergent (is finite). In the exercise you will prove that

2is a subspace of (N;C), and that 2is, in fact, an inner product space.

(a) Show that 2|ab| ≤ |a|2 +|b|2 ∀a,b ∈ C.

(b) Show that |a +b|2 ≤ 2|a|2 +2|b|2 ∀a,b ∈ C.

(c) Show that 2(N;C) is a subspace of (N;C).

(d) Show that u2:=∞n=1|un|2is a norm on 2(N;C).

Now consider the following mapping

〈u, v〉 :=

∞

i=1

ui vi where u, v ∈ 2(N;C). 35

(e) Show that 〈u, v〉 is well defined3for every u, v ∈ 2(N;C).

(f ) Show that this 〈·,·〉 is an inner product on 2(N;C).

(g) Show that this 〈·,·〉 is not an inner product on (N;C).

1.28 For which α ∈ R is

〈x, y〉 :=x1 y1 +α(x1 y2 + x2 y1)+ x2 y2

an inner product on R2?

1.29 Consider L 2([0, 1];R). Does a best approximation of etin the subspace of all polyno mial in L 2([0, 1];R) exist?

1.30 Does 2|〈x, y〉| ≤ x2 + y2 hold for any inner product?

1.31 Is  10x(t)y(t) dt + x(0)y(0) an inner product on C ([0, 1];R)?

1.32 Consider C ([0, 1];R). Show that none of the expressions below are inner products and indicate which of the axioms of inner product fail:

(a) 〈x, y〉 := 10x(t)y2(t) dt

(b) 〈x, y〉 := 101tx(t)y(t) dt

(c) 〈x, y〉 := 0.5

0x(t)y(t) dt

(d) 〈x, y〉 := 10 10x(t)y(s) dtds

(e) 〈x, y〉 :=x(0)y(0)+ x(1)y(1)

1.33 Consider C ([0, 1];R) with standard inner product. Determine the best approximation of t2in span{1,t}.

1.34 Consider C ([−1, 1];R) with standard inner product. Determine the best approximation of t4in span{1,t,t2,t3}.

1.35 Consider Cn. Let αk ∈ C. Show that

n

〈x, y〉 :=

k=1

αk xk yk

is an inner product if and only if all αk are real and larger than zero.

1.36 Show that {1/2,cos(πt),sin(πt),...,cos(nπt),sin(nπt)} is an orthogonal set on C ([−1, 1];R) with standard inner product.

1.37 Legendre polynomials. Consider C ([−1, 1];R) and its subspace P5([−1, 1];R) (the sub space of polynomials p : [−1, 1] → R of degree 5 or less.) It can be shown that the degree 6 Legendre polynomial is

p6(t) :=116(231t6 −315t4 +105t2 −5)

and that its norm is p6 = 2/13.

(a) Determine the best approximation of t6in P5([−1, 1];R).

3meaning that the infinite series i ui vi converges.

36

(b) Determine the minimal approximation error minq∈P5([−1,1];R) t6 − q.

(Hint: lengthy derivations can be avoided. The answer for part (b) is approximately 0.027.)

1.38 Construct an orthonormal basis of the space span{e−t, e−2t, e−3t} as subspace of L 2([0,∞);R) with standard inner product 〈f , g 〉 =  ∞

0f (t)g (t) dt.

1.39 Consider L 2([0, 1];R) with standard inner product, and let P ([0, 1];R) be the set of all polynomials in L 2([0, 1];R).

(a) Argue that the best approximation of etin the space of polynomials of any degree P ([0, 1];R) does not exist.

(b) Is the subspace P ([0, 1];R) a Hilbert space?

1.40 Consider the set of even square summable sequences

2,even = {x ∈ 2(Z,R)|xk = x−k }

and the standard inner product of 2(Z,R)

(a) Show that 2,even is a Hilbert space

(b) Determine a complete orthonormal system for 2,even

1.41 Show that finite with standard inner product (that of 2) is not a Hilbert space.

1.42 Suppose X and V ⊆ X are both Hilbert spaces. Show that every x ∈ X has a unique decomposition x = v + v⊥ with v ∈ V and v⊥ ∈ V⊥.

1.43 Show that 1and ∞ with standard norms are not inner product spaces.

1.44 Hilbert matrix. Let Pn−1 be the subspace of C ([0, 1];R) spanned by the monomi als {1,t,...,tn−1}. That is to say, Pn−1 is the subspace formed by the polynomials p(t) = α1 + α2t + ··· + αntn−1 of degree at most n − 1. We consider the standard norm, f 2 = 10f2(t) dt.

(a) Determine the normal equations (1.10) for the case that {v1,..., vn} = {1,t,...,tn−1} and x(t) = tn.

(b) Determine the best approximation of t2in P1.

1.45 Show that 〈f , g 〉 := 10f (t)g (t) + f(t)g(t) dt is an inner product on the vector space C1([0, 1];R) of continuously differentiable functions f : [0, 1] → R.

1.46 Can the arguments of Exercise 1.27(a,b) also be used to prove that L 2([a,b];R) is a sub space?

1.47 Is 1 ⊂ 2?

1.48 Let X be some complex Hilbert space.

(a) Let {vn}n∈N be an orthogonal sequence in X. Show that n∈N vn converges iff

n∈N vn2 < ∞.

(b) Let {en}n∈N be an orthonormal sequence in X, and αn ∈ C. Show that n∈N αnen converges iff α :=(α1,α2,...) is in 2(N;C).

37

(c) Let {en}n∈N be an orthonormal sequence in X, and α,β ∈ 2(N;C). Show that

〈 n∈N

αnen, n∈N

βnen〉X = 〈α,β〉2 .

Here 〈·,·〉X is the inner product on X, and 〈·,·〉2 the standard inner product on

2(N;C).

(d) Let {en}n∈N be an orthonormal sequence in X, and x ∈ X. Derive Bessel’s inequality:

n∈N

Tougher problems

|〈x,en〉|2 ≤ x2.

1.49 Norm or not a norm.

(a) Let µ ∈ [0, 1]. Show that every norm satisfies

µx +(1−µ)y ≤ µx +(1−µ)y.

(b) Consider R2. Given two elements x, y ∈ R2, say,

y

x

draw all elements of {z ∈ R2| z = µx +(1−µ)y,µ ∈ [0, 1]}.

(c) Show that z ≤ max(x,y) for every element of {z | z = µx +(1−µ)y,µ ∈ [0, 1]} (d) Consider R2. Can this set

be the unit ball {x | x ≤ 1} for some norm on R2?

1.50 Consider the sequence spaces 1,2,∞ and two of their continuous counterparts L 1,L 2. Prove or provide counter examples to the following claims:

(a) 2(N;R) ⊆ ∞(N;R) ?

(b) 1(N;R) ⊆ 2(N;R) ? [Hint: first assume that |xn| ≤ 1 for all n.]

(c) L 1([0, 1];R) ⊆ L 2([0, 1];R) ?

(d) L 2([0, 1];R) ⊆ L 1([0, 1];R) ? [Hint: first assume that |f (t)| ≥ 1 for all t.] (e) L 2(R;R) ⊆ L 1(R;R) ?

(f ) L 1(R;R) ⊆ L 2(R;R) ?

1.51 Consider two norms on Rn: an arbitrary norm · and the max-norm ·∞ (also known as ∞-norm). In this problem we derive that all norms on Rnare “equivalent” meaning that constants γ,δ > 0 exist (depending on n but not on x) such that

δx∞ ≤ x ≤ γx∞ ∀x. (1.15) 38

(a) Show that there is a γ > 0 such that

x ≤ γx∞.

[Hint: write x as x = x1e1 + ··· + xnen where {ei}i=1,...,n is the standard basis of Rn and express γ using the ei .]

(b) The Bolzano-Weierstraß theorem says that every bounded sequence in Rn has a convergent subsequence. This we can use to prove that a δ exists such that

δx∞ ≤ x.

To this end, let

δ0:= inf

x∞=1x.

If δ0 is zero then a sequence {xk } of vectors in Rnexists for which xk  → 0 as k → ∞. Now, by Bolzano-Weierstraß this means that a convergent subsequence {xkl} exists... Finish the proof, that is, prove that (1.15) holds.

1.52 Continuity of inner product. Let X be an inner product space and suppose that x :=limn→∞ xn exists in X. Use Cauchy-Schwarz to show that every inner product is continuous in the sense that limn→∞〈xn, y〉 = 〈x, y〉.

1.53 Chebyshev Polynomials. Let Tn : [−1, 1] → R be the Chebyshev polynomials Tn(t) = cos(n arccos(t)), n = 0, 1, 2,....

See Fig. 1.17. Soon we will see that the Tn(t) are indeed polynomials in t. In what follows Pn([−1, 1];R) denotes the set of polynomials p : [−1, 1] → R of degree n or less.

(a) Show that {Tn}n=0,1,2,... are orthogonal with respect to the inner product

〈f , g 〉 =

1

f (t)g (t)11− t2dt. (1.16) −1

(Hint: substitute t = cos(φ) and use the goniometric formula cos(t +s)+cos(t −s) = 2cos(t)cos(s))

(b) What is T0 and what is Tn for n > 0?

(c) Derive the recursion

Tn+1(t) = 2tTn(t)−Tn−1(t) ∀n ≥ 1,

and show that all Tn are polynomial. (Hint: again substitute t = cos(φ) and now use that cos(α)+cos(β) = 2cos(α+β

2)cos(α−β

2))

(d) Determine T0,T1,T2,T3.

(e) Use the previous part to determine the best approximation of t3in P2([−1, 1];R) with inner product (1.16).

(f ) Let n > 0. Determine the approximation error minq∈Pn−1([−1,1];R) tn − q in the norm defined by the inner product (1.16) (Hint: what is the leading coefficient of Tn?)

39

FIGURE 1.17: Graphs of the first six Chebyshev polynomials T0,...,T5 : [−1, 1] → R. (Exer cise 1.53)

1.54 Numerical solution. Verify that on C ([0, 1];R) with its standard inner product we have the recursion

〈tk, et〉 = e−k〈tk−1, et〉

for any k ∈ N. Use this and MATLAB or PYTHON (or whatever software) to find the opti mal degree 10 polynomial p(t) approximation of etand plot the error et − p(t).

1.55 Basis for images. Let e1 =10and e2 =01. This is the standard basis of R2. The standard basis for R2×2can be expressed in terms of e1,e2 as

1 0

0 0

= e1eT1,0 1

0 0

= e1eT2,0 0

1 0

= e2eT1,0 0

0 1

= e2eT2.

This suggests that we can construct orthonormal bases for Rn×kgiven any two bases for Rnand Rk. We analyze this situation not for not Rn×k but for “continuous square images”, that is,

L 2([0, 1]×[0, 1]) :={f : [0, 1]2 → C |

1 0

1 0

|f (t,s)|2dsdt < ∞}.

This space is a Hilbert space in the inner product

〈x, y〉 =

t=1 t=0

s=1 s=0

x(t,s)y(t,s) dsdt.

(a) Suppose {e1(t),e2(t),...} is an orthonormal basis of L 2[0, 1]. Show that φi j(t,s) :=ei(t)e j(s), i ∈ N, j ∈ N

is an orthonormal set on L 2([0, 1]×[0, 1]).

(b) Show that 〈x,φi j〉 = 0 for all i, j implies that x = 0 (this might be a tough problem). (c) Show that {φi,j}i,j∈N is an orthonormal basis of L 2([0, 1]×[0, 1]).

(d) Let φm,n(t,s) := ei2π(mt+ns). Is {φm,n}m,n∈Z an orthonormal basis of L 2([0, 1] × [0, 1])?

1.56 Example 1.5.5 shows that the even functions L 2even(R) in L 2(R) is a Hilbert space. Therefore, by Thm. 1.5.6, every f ∈ L 2(R) has a best approximation f∗ in L 2even(R). Show that

f∗(t) =f (t)+ f (−t)

2

is this best approximation.

40

1.57 Legendre polynomials & Rodrigues’s formula. Define the polynomials pn as

pn(t) =1 2nn!

dn(t2 −1)

dtn, n = 0, 1, 2,....

(a) Argue that pn has degree n.

(b) Show that {pn}n∈N is an orthogonal sequence on C ([−1, 1];R) with respect to the standard inner product.

[Hint: derive first that dn

dtn (t2 −1)n ⊥ 1,t,...,tn−1.]

(c) Show that (n +1)pn(t) = (2n +1)t pn(t)−npn−1(t) for all n ≥ 1. 41

42

Chapter 2

Introduction to Signals and Convolutions

f (t)

t

(a)

f (nTs)

n

(b)

FIGURE 2.1: A continuous-time signal and a discrete-time signal

2.1 Continuous-time and discrete-time signals

Quantities that change with time, such as the voltage across a resistor or the outdoor tem perature, may be regarded as functions f (t) defined on R or on a subset of R. Functions that depend on time are called signals. If a signal f (t) is defined for all t ∈ R or for all t in some interval (a,b) ⊂ R, then we say that f (t) is a continuous-time signal. If f (t) is defined only for a sequence of time instances

··· < t−2 < t−1 < t0 < t1 < t2 < ··· (tn ∈ R),

then f (t) is said to be a discrete-time signal. A typical example of discrete-time signal is a signal obtained through sampling of a continuous-time signal. The sampled signal of a continuous-time signal f (t) is the discrete-time signal f (nTs), n ∈ Z, defined at integer mul tiples of the sampling period Ts > 0. Figure 2.1(a) shows the plot of a damped sinusoid (continuous-time) and Figure 2.1(b) shows the corresponding sampled signal (discrete-time) for a certain sampling period. In plots, discrete-time signals are represented by a series of stems on the real axis, such as in Figure 2.1(b).

In this chapter we introduce a couple of standard signals and provide a classification of signals. A second purpose of this chapter is to review some mathematical techniques that we will need in the rest of this course.

43

−3+ i2

iy

i

−2 0 −i

z x

FIGURE 2.2: Several complex numbers in the complex plane

iy

φ

0

|z|

z

x

FIGURE 2.3: Modulus |z| and argument φ of a point z = x + iy in the complex plane C

2.2 Review of complex numbers

Signals in the real world take real values. Such signals are therefore called real signals or real-valued signals. For a comprehensive theory of signals and their transforms, however, we need complex signals. This section recaps a couple of results about complex numbers. This is a concise recap and is not suited as a first contact with complex numbers.

A complex number z may be expressed in canonical form,

z = x + iy, x, y ∈ R.

Here i is the imaginary unit which is a number having the property that i2 = −1.

The number x is known as the real part of z = x + iy, and we denote it as x = Re(z).

Likewise the imaginary part of x + iy equals y, notation

y = Im(z).

Note that the imaginary part is itself a real number (i.e. y ∈ R).

A complex number z may also be expressed in polar form,

z = r cos(φ)+ ir sin(φ), in which r ≥ 0 and φ ∈ R. (2.1)

Then r = |z| :=x2 + y2is the absolute value or modulus of z, and φ = arg(z) is the argument of z. The argument is the angle in radians that z makes with the positive real axis, and it is unique up to a multiple of 2π, see Fig. 2.3.

The set of complex numbers is denoted by C and it is commonly called the complex plane. The complex conjugate of z ∈ C will be denoted by z∗instead of the more common z¯. So if z = x + iy with x, y ∈ R then z∗ = x − iy, see Fig. 2.4.

44

0

z

z∗

FIGURE 2.4: The complex conjugate of a z ∈ C is denoted z∗and is obtained by swapping the sign of its imaginary part

Complex sum and product in canonical form

z1 + z2

z2

z1

0

FIGURE 2.5: Addition of two complex numbers in the complex plane

Sum of complex numbers. The sum of two complex numbers z1 = x1 + iy1 and z2 = x2 + iy2 is obtained by adding both real parts and both imaginary parts,

z1 + z2 = (x1 + x2)+ i(y1 + y2).

This corresponds to addition of vectors, see Fig. 2.5.

Product of complex numbers. To form the product of two complex numbers z1 = x1 + iy1 and z2 = x2 + iy2 we can work out the product using the rule that i2 = −1. So

z1z2 = (x1 + iy1)(x2 + iy2)

= x1x2 + x1iy2 + iy1x2 + i2

−1

y1 y2

= (x1x2 − y1 y2)+ i(x1 y2 + x2 y1).

Quotient of complex numbers. There is useful trick to find the canonical form of a quotient z1/z2:

z2=z1

z1

z2

z∗2

z∗2

=(x1 + iy1)(x2 − iy2) (x2 + iy2)(x2 − iy2)

1

=(x1x2 + y1 y2 + i(y1x2 − x1 y2) x22 + y22=

x1x2 + y1 y2 x22 + y22

+ i

y1x2 − x1 y2 x22 + y22

.

The following five rules are now readily verified.

|z∗| = |z|,

(z1 + z2)∗ = z∗1 + z∗2,

(z1z2)∗ = z∗1z∗2,

|z|2 = zz∗,

arg(z∗) = −arg(z).

45

(2.2)

Moreover, z∗ = z is a different way of saying that z is real.

The fundamental theorem of algebra

The familiar abc-formula states that the quadratic equation

az2 +bz +c = 0, (a,b,c ∈ R,a = 0)

has as solution

z1,2 =−b ±b2 −4ac

2a.

It may happen that b2 − 4ac < 0 in which case the quadratic equation has no real solutions. The formula however remains valid for complex numbers.

Example 2.2.1 (complex zeros). The two zeros of

z2 +2z +3

are

z1,2 =−2±−8

2= −1± i2.

Both zeros are complex.  The example demonstrates that while second degree polynomials need not have zeros over the real numbers z ∈ R, they always do have zeros over the complex numbers z ∈ C. More generally there is the famous result:

Theorem 2.2.2 (Fundamental theorem of algebra). Every non-constant polynomial p(z) = p0 + p1z +··· + pnzn(pi ∈ C) has a zero z ∈ C.

A consequence is that every polynomial of degree n ∈ N has precisely n zeros in the sense that for any such polynomial p(z), numbers λk ∈ C, k = 1, 2,...,n and c ∈ C,c = 0 exist such that

n

p(z) = c

k=1

(z −λk ).

Euler’s formula

The exponential function ez, with z ∈ C, can be defined as the series

ez:=1+ z +12!z2 +13!z3 +··· .

It is not hard to show that this series converges for every z ∈ C. Without proof we state that the familiar rule ea+b = eaebfor real a and b remains valid for a,b ∈ C. The exponential function is very interesting for imaginary z = iφ:

eiφ = 1+(iφ)+12!(iφ)2 +13!(iφ)3 +14!(iφ)4 + +15!(iφ)5 +···

1+12!i2φ2 +14!i4φ4 +··· +iφ+13!i3φ3 +15!i5φ5 +···

=

1−12!φ2 +14!φ4 +··· + iφ−13!φ3 +15!φ5 +···

=

= cos(φ)+ i sin(φ).

46

In the last equality we used the real Taylor series of cosine and sine. This connection is known as Euler’s formula:

i

−1

i sin(φ) −i

eiφ

φ

cos(φ)

1

FIGURE 2.6: Euler’s formula

Lemma 2.2.3 (Euler’s formula). eiφ = cos(φ)+ i sin(φ) for every φ ∈ R.  Since cos2(φ) + sin2(φ) = 1 we have that |eiφ| = 1. That is, eiφfor every real φ is an ele ment of the unit circle, see Fig. 2.6. As its real part is cos(φ) and imaginary part is sin(φ) it is immediate that arg(eiφ) = φ, see Fig. 2.6.

Euler’s formula expresses an exponential as a sum of cosine and sine, but it may also used to express cosine and sine as sums of exponentials. Indeed from

eiφ = cos(φ)+ i sin(φ)

e−iφ = cos(φ)− i sin(φ)

we readily get that

cos(φ) = Re (eiφ) =eiφ + e−iφ

2, (2.3)

sin(φ) = Im (eiφ) =eiφ − e−iφ

2i. (2.4)

A byproduct of Euler’s formula is that polar forms (2.1) can now be expressed more suc cinctly as

z = |z|ei arg(z).

This, in turn, opens up another way of forming products or quotients of complex numbers. For instance if z1 and z2 are given in polar form as

z1 = r1eiφ1, z2 = r2eiφ2

then their product in polar form simply is

z1z2 = r1eiφ1r2eiφ2 = (r1r2)ei(φ1+φ2).

The modulus of the product hence is the product of the moduli; on the other hand, the argu ment of the product is the sum of the arguments. The interpretation is that taking products in the complex plane means rotation about the origin. Similarly we have for quotients that

z2=r1eiφ1

z1

r2eiφ2= r1eiφ11r2e−iφ2 =r1r2ei(φ1−φ2). 47

zz2 1z2

φ1 +φ2 φ2

φ1

z1

FIGURE 2.7: The argument of a products z1z2 equals the sum of the arguments

1

FIGURE 2.8: The complex exponential signal e(−15+2i)tfor t ≥ 0 traces out this curve in the complex plane

2.3 Complex valued signals

The rules for sum, product and quotient also apply to complex-valued signals. For example, every complex signal f (t) can be expressed as

f (t) = A(t)eiφ(t),

where now modulus A(t) = |f (t)| and argument φ(t) = arg(f (t)) may depend on t. Example 2.3.1 (Exponential signals).

• An important example of a complex-valued signal is the harmonic signal f (t) = eiωt (see also Section 2.4), where ω is a real-valued constant. The modulus |f (t)| is equal to 1 for all t since

|eiωt| = |cos(ωt)+ i sin(ωt)| = cos2(ωt)+sin2(ωt) = 1 ∀t ∈ R.

• Let a = u + iv ∈ C, a = 0 and consider f (t) = e−at . Then

|f (t)| = |e−(u+iv)t| = |e−ut e−iv t |

= |e−ut ||e−iv t | = e−ut = e−(Re(a))t,

arg(f (t)) = arg(e−(u+iv)t) = arg(e−ut e−iv t)

= −v t = −Im(a)t,

f∗(t) = (e−(u+iv)t)∗ = (e−ut e−iv t)∗

= e−ut eiv t = e−(u−iv)t = e−a∗t.

48

• Let a ∈ C and suppose that Re(a) > 0. Then

t→∞e−at = 0.

lim

This is because its modulus |e−at | = e−Re (a)t decays exponentially to zero for Re (a) > 0 as t → ∞. As a consequence we have for every Re (a) > 0 that

∞ 0

M→∞

e−at dt = lim

e−at −a

M

=1a. (2.5) 0

• As t increases, the function

f (t) = e(−15+2i)t

traces out a curve in the complex plane that shrinks to zero (because Re (−15+ 2i) < 0) and that rotates counter-clockwise around the origin (because Im (−15+2i) = 2 > 0). See Fig. 2.8.

1

0

−2π

2π

ω

ω

−1

FIGURE 2.9: Graph of cos(ωt)

2.4 Periodic signals

A signal f (t) is periodic if a T > 0 exists such that f (t + T ) = f (t) for all t ∈ R. In that case T is known as a period of f (t). Be aware, however, that if T is a period then so is 2T and 3T etcetera. A signal will be referred to as T -periodic if it is periodic with period T .

An important class of real-valued periodic signals are the sinusoids or real harmonic sig nals.

Definition 2.4.1 (Sinusoids). A real-valued signal f (t) that can be written as f (t) = Acos(ωt +φ), A > 0, φ ∈ R, t ∈ R

is called a sinusoid or real harmonic signal. Then A is the amplitude, ω the angular frequency and φ the initial phase of the signal f (t).

It is easy to verify that a period of such sinusoids is T = 2π/ω. If the time t expresses seconds, then the angular frequency ω is in units of “radians” per second, and ω/(2π) is called the frequency and is in units of hertz (Hz). One hertz is one cycle per second. Similarly for the complex case we define the following.

Definition 2.4.2 (Harmonic signals). A signal f (t) that can be written in the form f (t) = ceiωt

with c ∈ C and ω ∈ R, is called a (complex) harmonic signal with amplitude |c|, angular fre quency ω and initial phase φ :=arg(c).

49

The connection between a real and a complex harmonic signal lies in Euler’s formula eiωt = cos(ωt) + i sin(ωt). Every linear combination of sinusoids with the same frequency, is again a sinusoid. Indeed, if a,b,ω ∈ R, then

a cos(ωt)+b sin(ωt) = aeiωt + e−iωt

2+beiωt − e−iωt

2i

=a − ib

2eiωt +a + ib

2e−iωt

= {write a − ib in polar form a − ib = Aeiφ}

=12Aeiφeiωt +12Ae−iφe−iωt

= Aei(ωt+φ) + e−i(ωt+φ)

2

= Acos(ωt +φ).

Note that complex harmonic signals are allowed to have a negative frequency. In the above we assumed that both sinusoids have the same frequency. That is crucial. If the frequencies differ, then the sum need not even be periodic. For example f (t) = cos(πt) + cos(t) is not periodic! In Chapter 3 we will see that practically every T -periodic signal can be written as a sum (possibly an infinite sum) of harmonic signals whose angular frequencies ω are integer multiples of 2π/T .

The next result expresses that the integral of a periodic signal over one period does not depend on were the interval over which is integrated is situated. This is a minor and intuitive result; it will be of use later.

Lemma 2.4.3. Suppose that f (t) is integrable on [0,T ] and that f (t) is periodic with period T > 0. Then for every a ∈ R, there holds

a+T a

f (t) dt =

T

f (t) dt.

0

a a +T 0 T

Proof. We write

a+T

f (t) dt =

a

0 a

f (t) dt +

T 0

f (t) dt +

a+T T

f (t) dt.

The result now follows because the first and third integral on the right-hand side cancel each other, which follows by substitution t = τ+T ,

a+T T

f (t) dt =

a 0

f (τ+T ) dτ =

a 0

f (τ) dτ = −

0

f (τ) dτ. (2.6) a

2.5 Standard signals

The following definition introduces four aperiodic signals that play a prominent role in this course.

Definition 2.5.1 (Four standard signals).

50

• For a given a > 0 the rectangular pulse recta(t) is defined as 1



0 if |t| > a/2,

recta(t) =

1

2if |t| = a/2



1 if |t| < a/2.−a2a2

In plots we normally use just one continuous line to represent the graph of the rectan gular pulse, even though the function is discontinuous (at t = ±a/2).

• For a given a > 0 the triangular pulse triana(t) is defined as



1

0 if |t| ≥ a,

triana(t) =

1−|t|aif |t| < a.−a a 

• The unit step 1(t) is defined as

1(t) =

 

0 if t < 0, 1

2if t = 0,

1

1 if t > 0.0 • The function sinc(t) (pronounced: “sink”) is defined as

sinc(t) =

 

sin(t)

tif t = 0,

1

1 if t = 0.π 2π

Note that at the jump discontinuities the function value is here taken to be the average of the function immediately before and after the jump, f (t) = (f (t−) + f (t+))/2. This choice is arbitrary but circumvents certain technicalities when Fourier series and Fourier integrals are considered.

Warning: In signal processing (and in MATLAB) the sinc function is defined as sinc(πt). It is a scaled version of our sinc, and it is zero at the nonzero integers (as opposed to the nonzero integer multiples of π).

We end this section with the definition of the class of piecewise smooth signals. These are signals that we will keep coming back to. The piecewise smooth signals encompass practically all continuous-time signals one is likely to come across in practice.

Definition 2.5.2 (Piecewise smooth signals). A signal f (t) is piecewise smooth on a finite interval [a,b] if a finite partition, a = c0 < c1 < c2 < ··· < cm = b, of the interval exists such that

1. f (t) is continuously differentiable at every t ∈ (ci,ci+1), (i = 0,...,m −1). 2. At every ci, (i = 1, 2,...,m −1) the following limits exist

f (c+i) := lim

h↓0f (ci +h), f(c+i):= lim

h↓0f(ci +h),

f (c−i) := lim

h↓0f (ci −h), f(c−i):= lim

h↓0f(ci −h),

51

3. f (a+), f (b−), f(a+) and f(b−) exist.

A signal f (t) defined for all t ∈ (−∞,∞) is said to be piecewise smooth if it is piece wise smooth on every finite interval [a,b]. The points ci are sometimes called the points of discontinuity, see Fig. 2.10.

c1 c2 c3 c4 t →

FIGURE 2.10: An example of a piecewise smooth signal

2.6 Energy and power

It is customary in signal analysis to use “energy” instead of norm:

Definition 2.6.1 (Energy content). The energy Ef of a signal f (t) is defined as

Ef =

∞

|f (t)|2dt.

−∞

If Ef < ∞ (finite energy content), then the signal is said to be an energy signal. The rect angular and triangular pulses are examples of energy signals. Sinusoids and harmonic signals are not. For example, the harmonic signal f (t) = ceiω0tsatisfies |f (t)| = |c| and, hence, Ef = ∞. For a signal f (t) to have a finite energy content it is necessary that limt→∞  ta|f (τ)|2 dτ converges. Consequently, signals like sinusoids, periodic signals and the unit step and many others, are not energy signals. In such cases it is customary to consider instead its averaged energy per unit time, i.e., to look at its (averaged) power.

Definition 2.6.2 (Power). The power Pf of a signal f (t) is defined as

Pf = lim M→∞

1

M

M/2 −M/2

|f (t)|2dt.

Signals that have finite power are called power signals. The power of a bounded T - periodic signal f (t) is finite, and you may wish to verify that its power then equals the average energy over one period,

Pf =1T T /2 −T /2

|f (t)|2dt if f (t) is T -periodic.

Example 2.6.3 (Power of a sinusoid). The power of the sinusoid f (t) = Acos(ω0t + φ) with period T = 2π/ω0, is

Pf =ω0 2π

π/ω0 −π/ω0

A2cos2(ω0t +φ) dt = {substitute x = ω0t} =A2

π

2π

−π

cos2(x +φ) dx =A22.

The last equality uses that cos2(α) =12+12cos(2α) which shows that the average of cos2(α) over a period is 12. This example assumes that ω0 > 0.

52

2.7 Convolutions

Loosely speaking a convolution is a linear combination of shifted copies of a signal. For in stance

3g (t)−41g (t −1)+501g (t +100) (2.7)

is an example of a convolution of a signal g (t). Note that the convolution is itself a signal. More generally expressions like

fτg (t −τ), fτ ∈ R (2.8) τ

are known as convolutions, and so is its integral version which we take to be its definition.

Definition 2.7.1 (Convolution). The convolution or convolution product of two signals f (t) and g (t) is denoted as (f ∗g )(t) and is defined as

∞

(f ∗g )(t) =

−∞

f (τ)g (t −τ) dτ.

It is an interesting fact that convolution products commute,

(f ∗g )(t) =

∞ −∞

f (τ)g (t −τ) dτ = {v = t −τ} =

∞ −∞

g (v)f (t − v) dv = (g ∗ f )(t).

In the following section, where we introduce the delta function, we will see that the sum (2.8) is a convolution as well (i.e. we can write it as this integral expression). Convolutions are very common in applications, and are, for instance, useful if we want to remove noise from signals, detect edges in pictures, soften pictures, etcetera.

u(t)

0 1

y(t)

(a)0 1

y(t)

(b)0 1

(c)

FIGURE 2.11: (a): A noisy signal; (b) averaged with P = 0.05; (c) averaged with P = 0.1

Example 2.7.2 (Sliding window averaging & noise reduction). For a given signal f (t) we construct the signal fswa(t) by averaging f (t) around t over an interval of a fixed length P, i.e., we consider

fswa(t) =1P t+P/2 t−P/2

f (τ) dτ.

Averaging f (t) this way filters out highly fluctuating noise. It is to be expected then, that fswa(t) is somewhat smoother than f (t), but as long as P is not too large the graph of the averaged fswa(t) should retain roughly the same shape as the graph of f (t). Figure 2.11(a) shows an example of a noisy signal f (t). Figure 2.11(b) shows fswa(t) for the case that P = 0.05. In plot (c) of that figure the average was taken over a wider interval (P = 0.1) and as expected the plot is smoother than the one in (b).

53

The signal fswa can be written as the convolution of f with a suitable function g :

f (t − v) dv = (f ∗g )(t),

for

fswa(t) =1P t+P/2 t−P/2

f (τ) dτ = {v = t −τ} =1P P/2

−P/2

1/P

g (t) =1PrectP (t)

−P

P

2

2

In order to determine a convolution it often is insightful to make plots of f (τ) and g (t −τ) as functions of τ. We demonstrate this on an example.

g (t −τ) f (τ)

t 0 τ →

g (t −τ) f (τ) 0 t τ →

FIGURE 2.12: f (τ) = e−τ1(τ) and g (t −τ) = e−2(t−τ)1(t −τ) for t < 0 (left) and t > 0 (right)

Example 2.7.3 (Visualizing convolution). Let f (t) = e−t1(t) and g (t) = e−2t1(t). Convolving f (t) and g (t) amounts to integration over all τ of the product of f (τ) and g (t −τ). To compute the convolution it is helpful to fix t and then examine the plots of f (τ) and g (t−τ) as functions of τ. Figure 2.12 shows two cases; one for t < 0 and one for t > 0. Clearly for t < 0 the nonzero parts of the functions f (τ) and g (t −τ) do not overlap, hence

∞

t < 0 =⇒ (f ∗g )(t) = −∞

f (τ)g (t −τ) dτ = 0.

If t > 0 then there is an overlap on the time interval [0,t]. Therefore

t > 0 =⇒ (f ∗g )(t) =

t 0

f (τ)g (t −τ) dτ =

t 0

e−τe−2(t−τ)dτ

=

t

e−2teτdτ = e−2teτt0 = e−t − e−2t. 0

With the unit step function we may combine the two cases t < 0 and t > 0: (f ∗g )(t) = (e−t − e−2t)1(t) ∀t.

Sufficient for the existence of (f ∗g )(t) is that f (t) is bounded while g (t) is absolutely in tegrable, or the other way around. Another important class of signals for which convolutions exist is the class of the so-called causal signals.

Definition 2.7.4 (Causal signal). A signal f (t) is causal if f (t) = 0 for all t < 0. (See Fig ure 2.13.)

54

0 t →

FIGURE 2.13: An example of a causal signal

A signal of the form f (t)1(t) is causal because 1(t) = 0 for t < 0. If f (t) and g (t) are causal, then

∞

(f ∗g )(t) =(f 1)∗(g 1)(t) =  ∞

f (τ)1(τ)g (t −τ)1(t −τ) dτ −∞

=

0

f (τ)g (t −τ)1(t −τ) dτ

=

0 if t < 0,  t

0f (τ)g (t −τ) dτ if t ≥ 0,

=

t 0

f (τ)g (t −τ) dτ

1(t).

The convolution of two causal signals apparently is itself causal, and since for each t the integration above is over a finite interval [0,t] it follows that the convolution exists for every t and every piecewise smooth f (t) and g (t).

Example 2.7.5 (Convolution with the unit step). Convolution with the unit step amounts to integration:

(f ∗1)(t) =

∞ −∞

f (τ)1(t −τ) dτ =

t

−∞

f (τ) dτ.

2.8 The delta function

FIGURE 2.14: A series of rn(t) for n = 1, n = 2, n = 3 and n = 4

In applications we often encounter signals that have a very short duration but neverthe less have a definite impact. An example is when you punch someone in the face (try it, it is fun). Such signals are called impulses. The standard impulse is the so-called Dirac delta function also known as the unit impulse. The delta function δ(t) is introduced as the limit as n → ∞ of

n

rn(t) :=

n if |t| < 12n

0 if |t| > 12n

−1

1

2n

2n

55

(2.9)

As n goes to infinity, the rectangular pulses rn(t) become spikier and spikier, with their spike around t = 0, see Figure 2.14. However, the area enclosed by the spike and the x-axis,  ∞

−∞ rn(t) dt, equals 1 independent of n. We now naively define the delta function δ(t) as the limit

δ(t) = limn→∞rn(t), (2.10)

and we think of the delta function as a “function” that is zero everywhere except at t = 0 where it has a spike so large that

0+

δ(t) dt = 1.

0−

The delta function is usually depicted as done in Figure 2.15, i.e., depicted by the zero func tion with a fat arrow pointing upwards at t = 0. The idea to see the delta function as a spike in this sense is helpful, but mathematically it is far from sound. After all,

limn→∞rn(t) =

0 if t = 0, ∞ if t = 0

and the integral of a function that is zero everywhere except for one point, is zero. Still, for many applications with delta functions it is enough to see δ(t) as the limit (2.10). Many tenta tive problems in calculations involving delta functions may be avoided if we stick to the rule the-last-limit-you-take. By that is meant that in calculations with delta functions, first δ(t) is replaced with the well defined rn(t) and only at the very last step the limit n → ∞ is taken. With this rule, for example, the following quintessential property of the delta function may be derived.

0 t →

FIGURE 2.15: The delta function δ(t)

Lemma 2.8.1 (delta function). If f (t) is continuous at t = 0, then

∞

δ(t)f (t) dt = f (0).

−∞

Proof. First replace δ(t) with rn(t),

∞ −∞

rn(t)f (t) dt = n

1/(2n) −1/(2n)

f (t) dt.

The integral is bounded from above by  1/(2n)

n

−1/(2n)

max

t∈[−12n,12n]f (t) dt = max t∈[−12n,12n]f (t)

and bounded from below by

1/(2n)

t∈[−12n,12n]f (t) dt = min

n

−1/(2n)

min

t∈[−12n,12n]f (t).

Now as n → ∞ the interval [−12n,12n] shrinks to zero and the two bounds converge to f (0). 56

n

t =ba−1

2n|a|

1

n|a|

t =ba+1

2n|a|

0 b

a

FIGURE 2.16: Shifted and scaled rn(t) 0 t = b t →

t →

FIGURE 2.17: Shifted delta function δ(t −b)

Properties of the delta function

Delta functions can be added, they can be multiplied with regular functions, they can be in tegrated etcetera. In this subsection we review the more important properties and rules for delta functions.

The scaled and shifted delta function δ(at − b) we take to be defined as δ(at − b) = limn→∞ rn(at − b). For t = b/a the argument at − b is zero, so rn(at − b) as a function of t is centered around t = b/a, see Fig. 2.16. This is very much like a shifted copy of rn(t) with the difference that the spike does not have a unit area. The width of the spike of rn(at − b) is easily seen to be 1/|an|, so the area of the spike is 1/|a|. In the limit as n → ∞ the spike therefore approaches 1/|a| times the delta function that has its spike at t = b/a:

δ(at −b) =1|a|δ(t −ba). (2.11) We can now generalize Lemma 2.8.1. If f (t) is continuous, then

∞ −∞

δ(at −b)f (t) dt =

∞ −∞

1

|a|δ(t −ba)f (t) dt (let τ = t −ba)

=1|a| ∞

−∞

An immediate special case is that  ∞

δ(τ)f (τ+ba) dτ =1|a|f (ba).

δ(t −b)f (t) dt = f (b), (if f (t) is continuous at t = b). (2.12) −∞

This property is known as the sifting property of the delta function. It is the property that out of all values {f (t) | t ∈ R} that f (t) can take, the value at t = b is sifted out. Note that δ(t −b) has its spike at t = b, see Figure 2.17, therefore another way to interpret the sifting property is that it says that  ∞

−∞ δ(t −b)f (t) dt equals f (t) at that t where δ(t −b) has its spike. It is also possible to determine the convolution product (f ∗δ)(t) of a signal f (t) and the delta function δ(t).

∞

(f ∗δ)(t) =

−∞

δ(t −τ)f (τ) dτ = {sifting property} = f (t). (2.13) 57

Here we used the fact that δ(t − τ) = δ(τ − t) as a function of τ has its spike at τ = t. A final useful property to have is the following.

Lemma 2.8.2 (Products with delta functions). If f (t) is continuous at t = b, then f (t)δ(t −b) = f (b)δ(t −b). (2.14)

Proof (idea). This is another instance were we will use the rule the-last-limit-you-take. The function f (t)rn(t −b) is zero for all |t −b| > 12n. On the interval [b−12n,b+12n] it generally has a spike. Since limn→∞  ∞

−∞ f (t)rn(t −b) dt = f (b) it means that the area of this spike approaches f (b) as n → ∞, so that f (t)rn(t) approaches f (b) times the delta function δ(t −b).

Example 2.8.3.

1. tδ(t) = 0×δ(t) = 0, i.e. the zero signal.

2. eiω0tδ(t −b) = eiω0bδ(t −b).

3. δ(−t) = δ(t). This is because of the scaling property for a = −1 (Table 2.1). 4.  t−∞ δ(τ) dτ = (1∗δ)(t) = 1(t) for all t = 0. So the step is an indefinite integral of δ(t).

TABLE 2.1: Properties and rules of calculus for the delta function

| Property |  | Condition |
| --- | --- | --- |
| Sifting - Convolution Scaling - | ∞ −∞ δ(t −b)f (t) dt = f (b) f (t)δ(t −b) = f (b)δ(t −b) (f ∗δ)(t) = f (t) δ(at −b) =1¦a¦δ(t −ba) t −∞ δ(τ) dτ = 1(t) | f (t) continuous at t = b f (t) continuous at t = b t = 0 |

Example 2.8.4 (Convolution with delta functions). Let

f (t) = 3δ(t)−41δ(t −1)+501δ(t +100)

Then using the sifting property of delta functions we get that

∞

(f ∗g )(t) =

−∞

[3δ(τ)−41δ(τ−1)+501δ(τ+100)]g (t −τ) dτ

= 3g (t)−41g (t −1)+501g (t +100).

This example demonstrates that linear combinations of shifted copies of g (t) are indeed con volutions as claimed in the previous section.

58

Generalized derivatives

Now that we have the delta function at our disposal we have a way of differentiating discon tinuous functions. Really! First an informal example.

Example 2.8.5. The unit step 1(t) may be seen as the limit as n → ∞ of

1

fn(t) =12+arctan(nt)

π.

(Convince yourself of this.) A clear difference between these fn(t) and their limit 1(t) is that all fn(t) are differentiable in the classical sense while their limit 1(t) obviously is not. Let us examine the derivatives of fn(t). These are

n(t) =1πn

n/π

f(1)

1+(nt)2.

And to what does it seem to converge as n → ∞? To a function that is zero everywhere except at zero where it is so huge that if we integrate over this peak we get

∞ −∞

f(1)

n(t) dt = fn(∞)− fn(−∞) = 1.

In other words the derivative f(1)

n (t) converges to the delta function as n → ∞. Now it is

tempting to claim that the derivative of 1(t) is δ(t). Weird.

It is actually not so difficult to make the claim of this example more precise. We simply redefine slightly what we mean with derivative. If two continuous signals f (t) and g (t) satisfy

t

f (t) = f (a)+ a

g (τ) dτ

for a certain a ∈ R or a = −∞, then we know that f (t) is continuously differentiable with derivative f(t) = g (t). The integral equality, however, may also hold for non-differentiable functions f (t), and, even in that case we will say that f (t) is differentiable and that g (t) is its (generalized) derivative, notation: f(t) = g (t). This generalization allows us to differentiate discontinuous functions.

Example 2.8.6.

• Let f (t) = |t| and consider the signal sgn(t) defined as

sgn(t) :=

 

1 if t > 0, 0 if t = 0, −1 if t < 0.

The function sgn(t) is the generalized derivative of f (t) = |t|, because  t

|t| =

0

sgn(τ) dτ.

• Let f (t) = 1(t). We showed earlier that  t

1(t) =

−∞

δ(τ) dτ.

So δ(t) is the generalized derivative of 1(t). Yes, we can differentiate discontinuous func tions now.

59

1

1

1

1

FIGURE 2.18: f (t) = 1(t)−1(t −1)+ e−αt1(t −1) and its generalized derivative f(t)

It may be shown that the product rule and chain rule of differentiation remain valid for derivatives in the generalized sense.

Example 2.8.7. Consider the signal f (t) depicted in Figure 2.18. It is the signal f (t) = 1(t)−1(t −1)+ e−αt1(t −1).

Its generalized derivative f(t) equals

f(t) = δ(t)−δ(t −1)−αe−αt1(t −1)+ e−αtδ(t −1)

= δ(t)−δ(t −1)−αe−αt1(t −1)+ e−αδ(t −1)

= δ(t)−(1− e−α)δ(t −1)−αe−αt1(t −1).

The generalized derivative f(t) is depicted in Figure 2.18(b).

We note that differentiation of a function at a point of discontinuity results in a delta func tion. If f (t) at t = b jumps from f (b−) to f (b+), then in the generalized derivative a delta function (f (b+)− f (b−))δ(t −b) shows up.

Example 2.8.8. The functions y(t) = e−t1(t) and u(t) = 1(t) are a (generalized) solution of the differential equation

y(t)+ y(t) = u(t)

because

y(t)+ y(t) =−e−t1(t)+ e−tδ(t)+ e−t1(t) = e−tδ(t) = δ(t) = u(t).

2.9 Exercises

2.1 Plot z1 and z2 in the complex plane, and then compute and plot both sum z1 + z2 and product z1z2 of

(a) z1 = 1+ i and z2 = 2+ i2,

(b) z1 = a + ib and z2 = b + ia for arbitrary a,b ∈ R.

2.2 Verify the equalities in Eqn. (2.2).

2.3 Show that limt→0e2it −1

t= 2i.

60

2.4 Does limt→∞ e−it2exist? Motivate your answer.

2.5 Show that limt→∞eitt= 0.

2.6 Demonstrate that

f (t) = 220sin(π25t)+110cos(π25t)

is a sinusoid, and find its amplitude.

2.7 Use Euler’s formula to prove that

sin(α)cos(β) =12sin(α+β)+12sin(α−β), ∀α,β ∈ R.

2.8 De Moivre: Show that cos(φ)+ i sin(φ)n= cos(nφ)+ i sin(nφ).

2.9 Demonstrate that

f (t) = sin(ω0t)+2cos(ω0t)−cos(ω0t +π/4)

is a sinusoid.

2.10 Let q ∈ Z. What is the smallest period of sin(2t)−sin(q t)?

2.11 A good understanding of a complex-valued signal f (t) can often be had from a plot of the curve (Re(f (t)), Im(f (t))) in the two-dimensional plane. Let ω0 = 0. Sketch by hand the curves in the complex plane

(a) f (t) = eiω0t.

(b) f (t) = eiω0t +0.1e10iω0t.

2.12 Calculate  2T

Te2iω0tcos2(ω0t) dt, where ω0 = 2π/T > 0.

2.13 Suppose T > 0 and define ω0 = 2π/T . Let f1(t) = sin(ω0t) and f2(t) = f1(t)1(t). Sketch the graphs of the two signals gi(t) (i = 1, 2) given by

t+T

gi(t) =

t

fi(τ) dτ.

2.14 Determine the power of the harmonic signal f (t) = 2e−2iω0t.

2.15 Let f (t) be a T -periodic signal with power Pf.

(a) Show that for every t0 ∈ R the power of f (t − t0) equals the power of f (t). (b) Express the power of f (2t) in terms of the power Pf of f (t).

2.16 Determine the energy content of

(a) f (t) = 2e−2iω0trecta(t).

(b) f (t) = rect2(t) trian2(t).

2.17 Show that for every t0 ∈ R the energy content of f (t − t0) is the same as that of f (t). 2.18 Show that the following signals are periodic, and determine their period.

61

(a) cos( 512 t)cos( 16t)

(b) cos( 23t)+sin( 47t)

2.19 Sketch the following signals.

(a) 1(t)−31(t −2)+21(t −5).

(b) trian2(t) rect1(t).

(c) e−t1(t).

(d) −et1(−t +1).

(e) sinc(t) rect4π(t).

(f ) sinc(π(t −4)).

2.20 Simplify the following expressions:

(a) sin(t)δ(t),

(b) e−5tδ(t),

(c) (t2 +2t)δ(t),

(d) (t2 +2t)δ(t −5),

(e) (t2 +2t)δ(t +5),

(f ) (f ∗g )(t) where f (t) = 1(t −5) and g (t) = δ(t),

(g) (f ∗g )(t) where f (t) = e5tand g (t) = δ(5t +5).

2.21 Given is a continuous function f (t).

(a) Let g1(t) = δ(2t +4). Determine (f ∗g1)(t).

(b) Let g2(t) = δ(2t −1). Determine f (t)g2(t).

2.22 Determine the derivative in the generalized sense of the following signals.

(a) t rect2(t),

(b) sin(t)1(t),

(c) t rect2(t −1),

(d) eit1(t −π),

(e) rect1(t) trian1(t).

2.23 Let f (t) and g (t) be two continuously differentiable signals. Determine the generalized derivative of f (t)1(−t)+ g (t)1(t).

2.24 Let f (t) = |sin(t)|. Compute the first and second order derivatives f(1)(t) and f(2)(t). 2.25 Determine the c,α ∈ R for which h(t) := ce−αt1(t) satisfies h(1)(t)+2h(t) = 10δ(t). 2.26 Determine the convolution (f ∗g )(t) for the following signals.

(a) f (t) = eat 1(−t) and g (t) = e−bt 1(t) with a > 0 and b > 0,

(b) f (t) = eat and g (t) = 1(t −1) (Re(a) > 0),

(c) f (t) = rect2(t) and g (t) = 1(t).

(d) f (t) = (1+ t) rect2(t) and g (t) = rect2(t).

62

2.27 Determine the convolution (f ∗g )(t) for the cases that

(a) f (t) = e−t1(t), g (t) = sgn(t),

(b) f (t) = 1(2t +1), g (t) = e−|t|,

(c) f (t) = g (t) = 1(t),

(d) f (t) = 1(t), g (t) = 1(t −1),

(e) f (t) = 1(t −1), g (t) = 1(t −1),

(f ) f (t) = e−t1(t), g (t) = et1(t),

(g) f (t) = eat 1(t), g (t) = ebt 1(t) with a,b ∈ C.

2.28 In the first part you will prove a property of the convolution which you will use in the second part.

(a) Prove that (f ∗(g +h))(t) = (f ∗g )(t)+(f ∗h)(t).

(b) Use the previous result and Euler’s formula to determine the convolution of f (t) = e−t1(t) and g (t) = cos(t)1(t).

More involved exercises

2.29 Determine a closed-form expression for Nn=1sin(nφ).

2.30 Let n ∈ N, n > 1, n even, and define z = eit. Show that sin(nt)

z−(n−1) = 2cos((n −1)t)+cos((n −3)t)+··· +cos(t).

sin(t)= zn−1+zn−3+···+z−(n−3)+

2.31 Dirichlet kernel. Consider the Dirichlet kernel1 defined as

N

DN (φ) = k=−N

eikφ.

(a) Is DN (φ) periodic?

(b) Show that for eiφ= 1 we have DN (φ) =e−iNφ − ei(N+1)φ

1− eiφ.

(c) Show that

DN (φ) =

2N +1 if φ is a multiple of 2π sin((N+1/2)φ)

sin(φ/2) if φ is not a multiple of 2π

= (2N +1)sinc((N +1/2)φ)

sinc(φ/2).

The plot of DN (t) is quite interesting, see Fig. 2.19. It consists of a series of peaks at integer multiples of 2π with oscillating behavior in between; oscillations that increase in number as N increases.

1Warning: other definitions of “Dirichlet kernel” exist, such as 12πDN (φ) and 1

2N+1DN (πt).

63

2N +1

−2π 0 2π φ FIGURE 2.19: The Dirichlet kernel DN (φ) for N = 8 (Exercise 2.31)

64

Chapter 3

Fourier Series

FIGURE 3.1: Jean Baptiste Joseph Fourier (1768–1830)

Why do we like Taylor series expansions such as

et = 1+ t +12!t2 +13!t3 +··· .

We like them because they break down complicated functions, here et, into a sum of simple functions tk. Ever wondered how calculators and programming languages calculate etand other functions? They rely on (a variation of ) Taylor series expansions. Indeed, computation of the terms in the Taylor series require only simple operations, so are readily computed. The message of this short story is this:

The point of expansions is

to break down complicated signals

into a series of simpler signals.

Now what is considered complicated or simple depends very much on the application. If the application is computation, then Taylor series expansions might be useful. However, in signal analysis the simple signals are the harmonic signals. There are very convincing arguments from systems theory which we can not go into here, but recall that harmonic signals are easy to generate (think of the power grid) and that rain decomposes visible light into a series of harmonic signals of different frequencies (the different colors of a rainbow).

65

Thus the questions is: which signals f (t) can be expanded into a series of harmonic sig nals, and how can we find these harmonic signals from f (t). That is the central question of this chapter. The discussion culminates in the famous result that says that practically every T -periodic signal f (t) can be expressed as a superposition1 of T -periodic harmonic signals,

f (t) = k∈Z

fk eik2πTt, fk ∈ C. (3.1)

The right-hand side is known as the Fourier series (expansion) of f (t). Note that the harmonic signals in the expansion indeed are all T -periodic:

eik2πT(t+T ) = eik2πTt+ik2π = eik2πTteik2π

1

If we define ω0 as

ω0 =2πT

= eik2πTt ∀t ∈ R.

then (3.1) is somewhat more succinctly described as

f (t) = k∈Z

fk eikω0t. (3.2)

It shows that the frequencies kω0 of the harmonic signals that together form f (t) are integer multiples of ω0. For this reason ω0:=2π/T is called the fundamental frequency of f (t).

3.1 The fundamental theorem of Fourier series

We momentarily return to the Hilbert space theory of Chapter 1. Let us first observe that the set of all complex harmonic functions with period T ,

ek (t) := ei2πTk t, k ∈ Z

is an orthonormal sequence in the complex inner product

〈f , g 〉 =1T T 0

f (t)g (t) dt.

This is readily verified: we have

ek 2 = 〈ek ,ek 〉 =1T T 0

and, if k = n,

ei2πTk t ei2πTk t

=1

dt = 1

〈ek ,en〉 =1T T 0

ei2πTk t ei2πTnt dt =1T T 0

ei2πT(k−n)tdt =ei2πT(k−n)t

T

i2π(k −n)

0

= 0.

So immediately we get that the best approximation f∗ in span{...,e−1,e0,e1,...} of a function f ∈ L 2([0,T ];C) is

∞

f∗ =

k=−∞

〈f ,ek 〉ek ,

1Superposition means sum.

66

that is,

f∗(t) =

with

∞

k=−∞

fk ei2πTk t

fk:=〈f ,ek 〉 =1T T 0

f (t)e−i2πk t dt.

The obvious question now is: is the Fourier series f∗ of f equal to f ? To put it differently, is the sequence of harmonic functions {...,e−1,e0,e1,...} a complete orthonormal sequence in L 2([0,T ];C)? It is:

h(t)

1

f (t)

0 1 c d

FIGURE 3.2: A shifted harmonic function h(t) so that h(t) ≥ 1 on [c,d] and 0 ≤ h(t) < 1 elsewhere on [0, 1]

Theorem 3.1.1 (Fourier series in L 2). Let T > 0. Every f ∈ L 2([0,T ];C) equals its Fourier series,

f (t) =

∞

k=−∞

fk eik2πTt. (3.3)

Here fk are the Fourier coefficients of f (t), defined as

fk = 〈f , eik2πT•〉 =1T T 0

f (t)e−ik2πTtdt. (3.4)

Proof. For ease of exposition we prove it only for the case T = 1. According to Theorem 1.5.7 it suffices to show that the only function f orthogonal to all ek is the zero function. Assume for the moment that f is real and continuous. If f is nonzero then f is > 0 (or < 0) throughout some small enough interval [c,d], see Fig. 3.2. Next let h(t) be the harmonic function

h(t) = b +12cos(2πt + a) = b +14(ei(2πt+a) + e−i(2πt+a))

where a,b are chosen such that h(t) ≥ 1 on the selected interval [c,d] and 0 ≤ h(t) < 1 else where, see Fig. 3.2. Realize that every power hn of h is a sum of harmonic functions, in fact

hn∈ span{e−n,...,e0,...,en}.

The higher the power, the larger hn(t) is in the interval [c,d] and the smaller it is elsewhere. Therefore for some large enough n we have

〈f ,hn〉 = 0.

67

This means that f can not be orthogonal to all ek . Hence the only continuous f that is or thogonal to all ek is the zero function, f = 0.

Now let f ∈ L 2([0, 1];R), not necessarily continuous, and assume that f ⊥ ek for all k ∈ Z. Again we show that f is then necessarily the zero function. Consider its antiderivative

t

F(t) :=

0

f (τ) dτ.

This function is continuous. It is also orthogonal to ek for all k = 0 because

〈F,ek 〉 =

e−i2πk tF(t) dt =e−i2πk t

1

−i2πkF(t)10−1

−i2πk〈f ,ek 〉 = 0 ∀k = 0.

0

Here we used that F(0) = 0 and F(1) = 10f (t) dt = 〈f ,e0〉 = 0. So F − 〈F,e0〉e0 is orthogonal to ek for all k ∈ Z, including k = 0. By continuity of F this implies that F − 〈F,e0〉e0 is the zero function, i.e. that F is a constant function. Its derivative f hence is the zero function, which is what we set out to prove.

For complex f ∈ L 2([0, 1];C) we can apply the arguments to both its real and imaginary parts.

Be aware that equality (3.3) is to be understood in L 2-sense! I.e. f (t) and its Fourier series f∗(t) might differ at some values of t, as long as they are the same for “almost all t” in the sense that f − f∗2 = 0. Using (3.4) we can compute the Fourier series from a given function.

T /2

0 T

t →

−T /2

FIGURE 3.3: Graph of the sawtooth of period T

Example 3.1.2 (Sawtooth). Figure 3.3 shows the graph of the sawtooth with period T . It is the T -periodic signal f (t) which on one period [0,T ) is given by

f (t) = t −T /2 if 0 ≤ t < T .

The Fourier coefficients of f (t) can be calculated explicitly from (3.4),

fk =1T T 0

f (t)e−ikω0tdt =1T T 0

(t −T /2)e−ikω0tdt.

Now we perform integration by parts to obtain, for k = 0, that

T

fk =−(t −T /2)e−ikω0t ikω0T

0

=−T /2e−ik2π −T /2

+1

ikω0T

T

T 0

e−ikω0tdt

ikω0T+1 ikω0T

0

e−ikω0tdt

=ikω0.

0

68

N = 1

N = 2

N = 4

N = 8

N = 16

FIGURE 3.4: The graph of the sawtooth (dashed) and of its truncated Fourier series N

k=−Nfk eikω0tfor N = 1, 2, 4, 8, 16

69

This assumes that k = 0. Remains to compute f0. It is defined as f0 =1T T /2

−T /2 f (t) dt which is

the average of f (t) over one period. The average of the sawtooth is zero,

f0 =1T T /2 −T /2

f (t) dt = 0.

Now we know all Fourier coefficients. Figure 3.4 shows plots of the partial sums Nk=−Nfk eikω0t for N = 1, 2, 4, 8, 16, and it seems that they indeed converge in some way to the sawtooth f (t) as N → ∞.

3.2 Pointwise convergence of Fourier series

Theorem 3.1.1 tells us that the Fourier series of f converges in L 2-norm to f whenever f ∈ L 2([0,T ];C). However, this does not imply that for every fixed t the partial sums

N

fk eikω0t

k=−N

converge (pointwise) to f (t) as N → ∞. In the example of the sawtooth the approximation is pretty good, except near the discontinuities of f (t). In fact, precisely at the points of discon tinuity the sawtooth, by definition, equals −T /2 whereas all truncated Fourier series are zero at these points! In this section we analyze pointwise convergence of Fourier series. We will soon see that the definition of convergence of Fourier series differs from that of other series. The following example motivates the change of definition.

Example 3.2.1 (From a real to a complex Fourier series). Suppose f (t) is a finite sum of real harmonic signals,

N

f (t) = k=0

ak cos(kω0t). (3.5)

This can be rewritten as a sum of complex harmonic signals as follows. N

f (t) = k=0

N

=

k=0

ak cos(kω0t)

ak

2(eikω0t + e−ikω0t)

N

= k=0

2eikω0t +0

ak

k=−N

a|k|

2eikω0t

N

= k=−N

fk eikω0t(3.6)

for f0:=a0 and fk:=12a|k|for all k = ±1,±2,...,±N.

The successive terms fk eiω0tgenerally are complex-valued functions of t, even if their sum is a real-valued function. Note that whereas the index k in the real case (3.5) goes from 0 to N, in the complex case (3.6) the index k goes from −N to N. Inspired by this we define pointwise convergence of Fourier series via symmetric limits:

70

Definition 3.2.2 (Pointwise convergence of complex Fourier series). Given t ∈ R, a Fourier series ∞k=−∞fk eikω0tis said to converge at t with limit f∗(t) if

N

f∗(t) = lim

fk eikω0t. (3.7)

N→∞

k=−N

When we write f∗(t) =∞k=−∞fk eikω0t we always mean (3.7). Note that the above is about pointwise convergence (at a given t). This is fundamentally different from convergence in L 2([0,T ];C) as considered in the previous section.

If in a Fourier series, f∗(t) =∞k=−∞fk eikω0t, only finitely many coefficients fk are nonzero, then obviously the Fourier series converges at every t ∈ R. More generally, pointwise convergence at every t ∈ R is ensured if the fk are absolutely summable, that is, if ∞k=−∞|fk | converges. In this case the sum f (t) is in fact continuous everywhere:

Theorem 3.2.3 (Existence of infinite Fourier series). If ∞k=−∞|fk | < ∞, then the Fourier se ries ∞k=−∞fk eikω0tconverges at every t ∈ R and is continuous at every t ∈ R.

Proof. Since |eikω0| = 1 it follows that |∞k=−∞fk eikω0t| < ∞k=−∞|fk | < ∞. We conclude that the Fourier series converges absolutely at every t, and, hence, that the Fourier series itself converges at every t. That the Fourier series is also continuous is shown2in Appendix A.1.

Hence if f∗(t) is not continuous then necessarily ∞k=−∞|fk | = ∞, and in that case the par tial sums sN (t) =Nk=−Nfk eikω0t may not provide a satisfactory approximation of f∗(t) near the points of discontinuity. A famous phenomenon in this respect is the Gibbs phenomenon discussed in § 3.6.

The following central result settles the issue of pointwise convergence. In this result we integrate over [−T /2,T /2] instead of [0,T ], but that is not relevant since all functions involved are T -periodic.

Theorem 3.2.4 (The Fourier series theorem). Let f (t) be a T -periodic signal and suppose it is piecewise smooth on R. Then for every t ∈ R there holds that

f (t+)+ f (t−) 2=

∞

k=−∞

fk eikω0t,

where fk are the Fourier coefficients of f (t) defined as

fk =1T T /2 −T /2

f (t)e−ikω0tdt. (3.8)

Proof. The proof is technical. A complete proof is given in Appendix A.1. If f (t) is, besides piecewise smooth, also continuous everywhere, then f (t+)+f (t−)

2clearly

equals f (t) everywhere, and hence the function and its Fourier series then are identical:

f (t) =

∞

k=−∞

fk eikω0t ∀t ∈ R.

2A nicer proof uses the fact that the partial sums sN (t) :=Nk=−Nfk eikω0tare Cauchy in C ([0,T ];C) with the max-norm, and this space is a Banach space (Thm. 1.2.2), so sN converges to a continuous function.

71

Example 3.2.5 (The sawtooth, continued). The sawtooth f (t) is continuous everywhere ex cept at t = 0,±T,±2T,.... At these values t = nT the signal satisfies f (nT +) = f (0+) = −T /2 and f (nT −) = f (0−) = T /2, see Fig. 3.3. According to the Fourier theorem, at these points of discontinuity the Fourier series equals (f (nT +)+ f (nT −))/2 = 0. The Fourier series therefore equals

k=0

i

kω0eikω0t =

0 if t = nT , n ∈ Z, f (t) elsewhere.

This is consistent with Fig. 3.4.

It is interesting to recall at this point Theorem 3.2.3. It says that ∞k=−∞|fk | < ∞ implies continuity of f (t). Now obviously the sawtooth is not continuous at t = nT , and, hence, we must have that ∞k=−∞|fk | = ∞. Indeed, ∞k=−∞|fk | = ∞k=12/|kω0| = ∞. The interpretation is this: for small values of k the terms fk eikω0t only change gradually as a function of time. So if a Fourier series is to approximate the abrupt change at a discontinuity of f (t) then it needs lots of high frequency components to accomplish this, i.e., the fk need to be relatively large for large values of k, so large in fact that ∞k=−∞|fk | diverges to ∞.

Even though ∞k=−∞|fk | might diverge, we always have that Fourier coefficients fk of piecewise continuous functions f (t) converge to zero as k → ∞. This is a byproduct of the Riemann–Lebesgue lemma:

Lemma 3.2.6 (Riemann–Lebesgue). If f (t) is piecewise smooth on [a,b], then

lim

|ω|→∞

b a

f (t)eiωtdt = 0.

Proof. Suppose first that f (t) is continuously differentiable on [a,b]. Then we can use inte gration by parts to obtain

b a

f (t)eiωtdt =1iωf (t)eiωtba−1iω b a

f(t)eiωtdt.

Since |eiωt| = 1 we can derive from this the bound

b

a

f (t)eiωtdt

≤1|iω|(|f (b)| +|f (a)|)+1|iω| b a

|f(t)|dt.

It is immediate that the right-hand side goes to zero as |ω| → ∞, which proves the claim. If f (t) is not continuously differentiable, then, since f (t) is piecewise smooth, we may split [a,b] into a finite set of subintervals [ti,ti+1] (i = 1,...) such that f (t) is continuously differentiable on each of these subintervals. Similarly as done above (using integration by parts) it follows that lim|ω|→∞  ti+1

tif (t)eiωt dt = 0. Hence lim|ω|→∞  baf (t)eiωt dt = 0.

An immediate consequence of the Riemann-Lebesgue lemma is that piecewise smooth signals f (t) satisfy

lim

|k|→∞

T /2 −T /2

f (t)e−ikω0tdt = 0,

i.e., the Fourier coefficients fk converge to zero as |k| → ∞. In fact, looking at the proof of the Riemann–Lebesgue lemma, we may conclude that |fk | ≤ A/|k| for some A (depending on f but not on k).

72

Remark. There is also a Fourier series expansion for two-dimensional signals g (x, y), i.e., images, and like f (t) may be represented by its Fourier coefficients, also pictures may be rep resented by Fourier coefficients. Simply discarding high frequency Fourier coefficients then gives a finite set of Fourier coefficients that roughly capture the original signal or picture. This is the essence of jpeg-encoding. (It also explains why jpeg pictures typically are softer than the original: sharp edges need high frequency components but these are discarded in the encoding.)

3.3 Real Fourier series

For real-valued signals f (t) the Fourier coefficients obey the symmetry rule f−k = f∗k. This follows from

f−k =1T T /2 −T /2

f (t)eikω0tdt = {f (t) is real} =1T T /2 −T /2

f∗(t)eikω0tdt

=

1

T

T /2 −T /2

f (t)e−ikω0tdt

∗

= f∗k.

The complex Fourier series of a real-valued function can then be rewritten as a real Fourier series, that is, as a sum of real sinusoids. This goes as follows. First define real ak and bk via

ak = 2Re fk , bk = −2 Im fk ∀k = 0, 1, 2,.... (3.9) That is to say

f0 =a02, fk =ak − ibk

2∀k > 0.

This rather odd looking definition of ak and bk will turn out to be useful. Then

∞

k=−∞

fk eikω0t = f0 +∞ k=1

(f−k e−ikω0t + fk eikω0t)

= {f−k = f∗k} = f0 +∞

k=1

(fk eikω0t)∗ + fk eikω0t

= f0 +

∞

Re(2fk eikω0t). (3.10) k=1

=12a0 +∞ k=1

Re(ak − ibk )

2fk

(cos(kω0t)+ i sin(kω0t))

eikω0t

=12a0 +∞

k=1

ak cos(kω0t)+bk sin(kω0t). (3.11)

This establishes that f (t) is indeed a sum of sinusoids. It will be clear that any f (t) that is of the form (3.11) is a real-valued T -periodic signal with T = 2π/ω0. The series (3.11) is known as the real Fourier series and the coefficients ak and bk are the real Fourier coefficients. The term ak cos(kω0t)+bk sin(kω0t) is sometimes referred to as the k-th harmonic of f (t). Sum mary:

Theorem 3.3.1 (Fourier series theorem, real-valued case). Let f (t) be a real-valued T - periodic signal and suppose it is piecewise smooth. Then for every t ∈ R we have

f (t−)+ f (t+)

2=12a0 +∞

k=1

ak cos(kω0t)+bk sin(kω0t), 73

where ak and bk are the real Fourier coefficients of f (t) defined as

ak =2T T /2

−T /2

bk =2T T /2 −T /2

f (t)cos(kω0t) dt, k = 0, 1,..., (3.12) f (t)sin(kω0t) dt, k = 1, 2,.... (3.13)

Proof. Let fk be the complex Fourier coefficients of f (t). In the above we showed that ak = 2Re fk and bk = −2 Im fk . Therefore

f (t)cos(kω0t) dt,

and

ak = 2Re fk = 2Re1T T /2 −T /2

f (t)e−ikω0tdt =2T T /2 −T /2

bk = −2Im fk = −2Im1T T /2 −T /2

f (t)e−ikω0tdt =2T T /2 −T /2

f (t)sin(kω0t) dt.

Example 3.3.2 (Sawtooth, real Fourier series). In Example 3.1.2 we found that the Fourier coefficients of the saw tooth of Fig. 3.3 are

f0 = 0, fk =ikω0∀k = 0.

Hence the real Fourier coefficients are

ak = 2Re (fk ) = 0, bk = −2 Im(fk ) = −2

kω0= −2k2πT= −Tkπ.

Surprisingly perhaps, all ak are zero, so the real Fourier series consists of sines only,

f (t−)+ f (t+)

2=

All coefficients −T

∞

k=1

−T

kπsin(kω0t).

kπare negative. This is consistent with Fig. 3.4.

It is not uncommon that Fourier series only consists of sine terms (or only cosine terms). This is due to symmetry properties that signals f (t) often have. The sawtooth is an odd signal.

Definition 3.3.3 (Even and odd signals). A signal f (t) is odd if

f (t) = −f (−t) ∀t.

A signal f (t) is even if

f (t) = f (−t) ∀t.

Another way to think of even and odd is that the graph of an even signal is symmetric with respect to the ‘y-axis’ whereas odd means it is point-symmetric with respect to the origin. Clearly cosines are even and sines are odd. It is then no surprise (and easily proved in general, see the next example) that Fourier series of odd signals, such as the sawtooth, only consist of sines, and Fourier series of even signals only consist of cosines.

74

FIGURE 3.5: f (t) = |sin(πt)|

Example 3.3.4 (Real Fourier coefficients). Consider the even signal f (t) = |sin(πt)|, (see Figure 3.5). This signal is periodic with period T = 1. This signal is even, but then all gk (t) := f (t)sin(kω0t) are odd. Hence the integrals for bk ,

bk =2T T /2 −T /2

f (t)sin(kω0t)

gk (t)

dt, k = 1, 2,...

are all zero. The Fourier series of f (t) entails only cosines.

ak =2T T /2 −T /2

f (t)cos(kω0t) dt = 2

1/2 −1/2

|sin(πt)|cos(2kπt) dt

= { integrand is even, hence  y−y = 2 y0}  1/2

= 4

0

sin(πt)cos(2kπt) dt

= { sin(α)cos(β) =12sin(α+β)+12sin(α−β), see Exercise 2.7 },  1/2

= 2

0

(sin((2k +1)πt)−sin((2k −1)πt)) dt

=−2cos((2k +1)πt) (2k +1)π

=2π1

1/2 0

−−2cos((2k −1)πt) (2k −1)π

1/2 0

2k +1−1

2k −1

=4π1

1−4k2.

The signal f (t) is continuous and is piecewise smooth on R, so we conclude that

|sin(πt)| =2π+4π∞ k=1

1

1−4k2cos(2kπt)

for every t ∈ R. Funny formula. For t = 0 it says that ∞k=11

1−4k2 = −1/2. For t = 1/2 it also

gives a funny formula (try it yourself ).

Time and frequency domain. According to the Fourier theorem, any piecewise smooth pe riodic signal f (t) may be reconstructed from its Fourier coefficients fk , except at its points of discontinuity. Possibly after redefining f (t) at the discontinuities, one can say that f (t) is fully determined once its Fourier coefficients are known.

We say that fk describes f (t) in the frequency domain, and that f (t) itself describes the signal in the time domain.

We sometimes refer to the fk as the line spectrum of f (t). Line spectra are usually com plex numbers. These may be expressed in polar form as fk = |fk |eiφk. The plot of |fk | ver sus frequency index k (or versus frequency kω0) is referred to as the amplitude spectrum of f (t), and the plot φk versus k (or versus kω0) as the phase spectrum of f (t). The phase for a nonzero coefficient, is unique up to an integer multiple of 2π. For zero coefficients we often set the phase equal to zero.

75

If f (t) is real-valued, then f−k = f∗k, hence, |fk | = |f−k | and φk = −φ−k . Real-valued signals hence have an amplitude spectrum that is even and a phase spectrum that is odd (up to multiples of 2π), see Fig. 3.6.

|fk |

0 1 2 k →

arg(fk)

0 1 2 k →

FIGURE 3.6: Amplitude spectrum (top) and phase spectrum (bottom) of the sawtooth, see Example 3.3.5

Example 3.3.5 (Amplitude and phase spectrum). In Example 3.1.2 we found the Fourier co efficients of the sawtooth to be fk = i/(kω0) for k = 0 and fk = 0 for k = 0. The amplitude of the Fourier coefficients is therefore |fk | = 1/|kω0|, except for k = 0 where it is zero. The phase φk = arg fk equals π/2 for positive k, and −π/2 for negative k. The phase of f0 = 0 is not really defined, but in such cases we take it to be zero. Figure 3.6 shows the amplitude and phase spectrum of the sawtooth (for a certain ω0). The sawtooth is a real-valued function and this is in accordance with the fact that the amplitude spectrum is an even function of k and that the phase spectrum is an odd function of k.

3.4 Fourier series properties

It is convenient to express the connection between a function f (t) and its Fourier coefficients fk as a pair with arrows:

f (t) ←→ fk .

Table 3.1 collects a number of properties of Fourier series. We discuss them here. Linearity. The linearity property says that for every two constants α,β we have αf (t)+βg (t) ←→ αfk +βgk .

This is a consequence of linearity of integration: the Fourier coefficients dk of d(t) = αf (t)+βg (t) equals

dk =1T T /2 −T /2

(αf (t)+βg (t))e−ikω0tdt

= α1T T /2 −T /2

f (t)e−ikω0tdt +β1T T /2 −T /2

76

g (t)e−ikω0tdt = αfk +βgk .